# LAST NIGHT — Спецификация продукта

Постапокалиптический ревербератор холодной ночи. Сменные физические пластины, дыхание руин, тиканье Гейгера, голос мёртвой электросети, колокол в опустевшем соборе.

Версия: v5.0 — hybrid lock (Decision 09): visual canon = mockup пользователя, electrical innovations = Decision 08 Часть серии: SYSTEM SUICIDE, модуль 9 из 9 — combine "холодной ночи", симметричный диптих с Last Day "раскалённого полудня"

Форм-факторы (две canonical SKU, общая PCB, разные корпуса):

- Eurorack 3U × 40HP (203.2 × 128.5мм panel) — для modular setup.

- Big-box pedal ~203×140мм (Strymon BigSky / Eventide H9 Max class) — для pedalboard.

Цена retail:

- Eurorack 40HP: $499 budget / $649 premium.

- Pedal big-box: $499 budget / $649 premium (одинаковая, разный enclosure).

Phase plan: Phase 1 ship = reverb-ядро + phaser + noise/geiger секция + 4 footswitches (TAP/GATE-CRUSH/BYPASS/FREEZE). Cold palette FX (PULSE/FOG/FROST/CHILL/HUM) — Phase 2 v3 PCB upgrade kit. Solenoid double-function (TOLL strike + STALL hold) — modular-advanced feature, доступна через CV-only triggers (J_TOLL_TRIG, J_STALL_CV), не через footswitches.

## Содержание

- Обзор

- Архитектура и signal flow

- Контролы и интерфейс

- Картриджи — каталог материалов

- Тембральный характер

- Сценарии использования

- Технические характеристики

- Совместимость и patching

- Уход и обслуживание

## Обзор

### Что это

Last Night — постапокалипсис-ревербератор на сменных физических пластинах, выпускается в двух canonical SKU (40HP Eurorack module и big-box pedal). Каноническая комбайн-форма "холодной ночи" в серии SYSTEM SUICIDE, симметричный диптих с Last Day "раскалённого полудня".

Реальная пластина из дерева, камня, металла, стекла или кости вибрирует от audio signal через surface exciter, два пьезо-датчика снимают резонанс. Каждый материал — уникальный голос: oak = тёплый slap, marble = cathedral, spring steel = infinite shimmer, glass = crystalline chime, bone = ritual.

Поверх ядра наслоены характерные эффекты "холодной палитры": NOISE + COLOR(geiger) — 2 отдельные ручки (NOISE = level, COLOR = continuous crossfader от continuous zener hiss к Geiger cluster ticks), always-on phaser (Phase/Flutter knob) с analog Function Generator для LFO (rise/fall/depth sliders + exp/log + speed/range knobs + 4 outputs EG/Gate/Sub÷2/Inv) — FG primarily modulates phaser но output patchable куда угодно через CV bay. Performance gestures: TAP / GATE-CRUSH / BYPASS / FREEZE — четыре footswitches. Solenoid triple-function (DAMP envelope CV + TOLL impulse + STALL hold) доступна как modular-advanced features через CV-only triggers (J_TOLL_TRIG normalled от FG EOR, J_STALL_CV). В v3 PCB revision (Phase 2 upgrade) добавляются cold-palette эффекты: PULSE (slow damper-breath), FOG (apperiodic drift), FROST (HF absorber), CHILL (anti-compression), HUM (mains-hum antenna).

### Чем отличается от обычных pedal-reverb

- Сменные материальные картриджи — никто другой не делает physical-plate reverb с swappable physical resonators.

- Solenoid double-function — один соленоид, два режима: DAMP (sustained hold = мех. приглушение хвоста) и TOLL (5-10мс impulse strike = пластина звенит как колокол независимо от input).

- Dual piezo с position crossfade — два pickup-а на пластине, manual или CV blend.

- NOISE + COLOR(geiger) — 2 separate knobs: NOISE = output level (zener + LFSR shared internally), COLOR = continuous crossfader от continuous zener hiss к Geiger cluster ticks (ATtiny85 LFSR + comparator pattern). Два sonic universe через 2 ручки (mockup canon).

- Always-on phaser — analog 4-stage OTA, named effect. Phase/Flutter knob = continuous feedback intensity morph (subtle → resonant → controlled self-oscillation). LFO source = analog Function Generator с 3 sliders (rise/fall/depth) + 2 knobs (exp/log curve, speed/range). FG outputs exposed via 4 jacks (EG/Gate/Sub÷2/Inv). Bypass через footswitch BYPASS.

- Cold-palette FX layer (Phase 2) — PULSE/FOG/FROST/CHILL/HUM — symmetric antipodes к hot-palette Last Day (HAZE/MIRAGE/BLEACH/TAR/CICADA/HEATWAVE).

- Modular-style CV patch bay — 14+ CV inputs для full automation.

- Stereo + dry/wet split outputs + TRS/RCA параллельно + instrument/-10/+4dBu level switch.

### Категория

Resonator reverb combine ("комбайн холодной ночи"). Не имитация EMT 140 plate reverb, не digital algorithmic. Новая категория — physical resonance + cold-palette FX layer.

Конкуренты в premium boutique tier:

- Strymon BigSky — DSP reverb с modes, $499.

- Eventide H9 Max — multi-effects, $679.

- Hologram Microcosm — granular/loop pedal, $399.

- Empress Reverb — algorithmic, $499.

- Strymon NightSky — DSP с CV, $400.

- Chase Bliss Habit — long-loop processor, $399.

Last Night sits в premium boutique tier ($499–649) с unique physical differentiator (cartridge swap + solenoid double-function).

### Reality check (для customer expectations)

Заявленные в первоначальной концепции RT60 ("oak 1.5–4с") завышены в 2–20×. Реальные значения:

- Oak без feedback: 0.1–0.3с (perкуссивный slap).

- Marble: 0.8–2с (room ambience).

- Spring steel: 2–6с (long shimmer).

- С feedback knob — RT60 удлиняется до self-oscillation drone.

- С phaser + cold-palette FX поверх — perceived "size" звука увеличивается ещё в 2–3× за счёт modulation tail.

### Use cases

- Guitar/bass pedal в pedalboard chain — после fuzz, перед delay/looper.

- Synth processor — для desktop synth setup, заменяет Eurorack reverb.

- DJ FX send/return — line-level routing через RCA параллельно с TRS.

- Ambient sound design — freeze + high feedback + phaser depth + slow PULSE damper-breath.

- Noise / experimental — NOISE + COLOR (geiger) full CW + self-oscillating cartridge + GATE-CRUSH footswitch + TOLL bell-strike via CV trigger (J_TOLL_TRIG).

- Ritual perform — TOLL bell-pattern (CV from sequencer) + STALL hold (J_STALL_CV) + cartridge swap drama + FREEZE footswitch для loop sustain.

- Vocals / drums (studio) — line-level через Hi-Z input compatible.

### Эффекты "холодной палитры" (cold-palette FX layer)

Постапокалипсис холодной ночи — словарь эффектов, каждый имеет физическое основание и парный антипод в Last Day:

Phase 1 ship (ядро + базовые FX, mockup canon):

- NOISE + COLOR(geiger) — 2 separate knobs: NOISE = output level (после shared crossfader), COLOR(geiger) = continuous crossfader от continuous zener hiss (CCW, BZX55C9V1) к Geiger cluster ticks (CW, ATtiny85 LFSR + comparator pattern). Internally один shared generator + OTA crossfader pair.

- Phaser always-on + analog Function Generator — 4-stage OTA all-pass (LM13700 U7+U8). Phase/Flutter knob = single continuous feedback intensity morph. LFO source = analog FG (Tides-class): rise/fall/depth sliders shape waveform, exp/log knob morphs curve, speed/range knob + clock sync set rate. 4 FG outputs (EG main, Gate rise-active, Sub÷2, Inv) exposed via panel jacks для patch-to-anything routing. Bypass через footswitch BYPASS.

- Gate / Crush — footswitch destruction: 4066 CMOS gate + LF398 sample-hold bitcrush.

- Solenoid triple-function через CV-only triggers (modular-advanced features): DAMP (J_CV_DAMP envelope CV), TOLL (J_TOLL_TRIG gate → 555 monostable → 5–10мс strike pulse), STALL (J_STALL_CV sustained hold).

Phase 2 ship (cold-palette upgrade kit, v3 PCB revision):

| Cold (Night) | Hot antipode (Day) | Физика |
| --- | --- | --- |
| PULSE | HAZE | Slow LFO 0.05–2Гц → damper-pressure → "tail дышит" ритмично |
| FOG | MIRAGE | Apperiodic drift: 4066 S&H → slow slew → damper micro-mod |
| FROST | BLEACH | Voltage-controlled LPF в feedback path → sweep-down ВЧ |
| CHILL | TAR | Expander с brittle release: anti-compression, quiet stays quiet |
| HUM | HEATWAVE | Ferrite coil 50/60Hz antenna → tuned amp → mix |

Каждый — knob на лицевой панели в v3 revision (Phase 2 upgrade kit).

### What's in the box

Eurorack SKU (40HP):

- Last Night module (203×128.5мм aluminum panel + 190×108мм PCB).

- 1× wood cartridge (Oak raw, reference).

- 1× shielded cartridge cable (mini-XLR + JST internal).

- Eurorack ribbon power cable (10/16-pin).

- Quick start card + warranty card.

Pedal SKU (big-box class — Strymon BigSky / Eventide H9 Max):

- Last Night pedal (anodized aluminum corpus 203×140×40мм).

- 1× wood cartridge (Oak raw, reference).

- 1× shielded cartridge cable (mini-XLR + JST internal).

- 12V DC center-negative pedal supply (regulated, 500mA min) — отдельно или в premium bundle. Compatible: Voodoo Lab Pedal Power 4×4 (12V output), Cioks DC10, Eventide PowerMax.

- Quick start card + warranty card.

Обе SKU делят: identical schematic, identical cartridge spec, identical sound. Cartridges interchangeable между Eurorack и pedal версиями.

### Compatibility — Eurorack ↔ Pedal

Eurorack SKU:

- Power: ±12V bus, 200mA steady / 450mA peak.

- I/O: 3.5мм TS Thonkiconn jacks (audio + CV).

- Mount: 3U × 40HP standard rack.

Pedal SKU:

- Power: 12V DC center-negative (современный стандарт для complex pedals — Strymon, Eventide, Meris, Chase Bliss). Internal isolated DC-DC (TRACO TMR 3-1212WI или Recom RKD-1212) → bipolar ±12V audio rails. 500mA min supply current.

- Identical audio headroom к Eurorack version (±12V rails везде, same +15 dBu max output).

- Isolated DC-DC также ломает ground loops с другими pedals на pedalboard — clean integration в любой rig.

- I/O: 6.3мм TS jacks (main audio in/out), 3.5мм mini-jack (CV expansion patch bay).

- Bypass: relay-buffered true bypass (no degradation when off).

- Dimensions: big-box class (203×140×40мм) — Strymon BigSky / Eventide H9 Max class.

- Weight: ~500г.

### Cross-compatibility — single instrument, two enclosures

- Identical schematic: same blocks, same component values, same calibration.

- Identical cartridges: купленный к Eurorack картридж работает в pedal (и наоборот).

- Identical sound: бит-в-бит идентичный reverb + always-on phaser + cold palette FX character.

- Identical price: $499 budget / $649 premium для обеих SKU.

- Different power, panel, connectors — только enclosure-level отличия.

Customer может start с pedal (для studio/live performance), позже добавить Eurorack для modular setup без потери tone.

## Архитектура и signal flow

### High-level signal chain (с FX engine)

┌────────────────────────────────────────────────────────────────────────────┐
│                                                                            │
│   IN ─► Buffer ─► [Pre-FX route] ─► Pre-emph ─► Driver ─► EXCITER ─┐      │
│           │                          (shelf)     (class AB)         ▼      │
│           │                                                  [PHYSICAL     │
│           ├── DRY path ──────────────────────────────┐        PLATE]       │
│           │                                          │           │         │
│           │                                          │           ▼         │
│           │                                          │   ┌── PIEZO A      │
│           │                                          │   ├── PIEZO B      │
│           │                                          │           │         │
│           │                                          │   JFET preamp      │
│           │                                          │   (LSK489A ×23)    │
│           │                                          │           │         │
│           │                                          │   POSITION xfade   │
│           │                                          │           │         │
│           │                                          │   De-emph ─► HiPass│
│           │                                          │           ► LowPass│
│           │                                          │           │         │
│           │                                          │   ┌───────┴────────┐│
│           │                                          │   │  PHASER        ││
│           │                                          │   │  (4-stage OTA, ││
│           │                                          │   │   Phase/Flutter││
│           │                                          │   │   DEPTH/SPEED) ││
│           │                                          │   └───────┬────────┘│
│           │                                          │           │         │
│           │                                          │   ┌───────┴────────┐│
│           │                                          │   │  VINYL FX      ││
│           │                                          │   │  (wow/flutter, ││
│           │                                          │   │   pitch warp,  ││
│           │                                          │   │   shape form)  ││
│           │                                          │   └───────┬────────┘│
│           │                                          │           │         │
│           │                                          │      LED clipper   │
│           │                                          │           │         │
│           │                                          │      ENV VCA       │
│           │                                          │      (LM13700)     │
│           │                                          │           │         │
│           ├──────────────────────────────────────────┴────► MIX ─┤         │
│           │                                                       │         │
│   Noise gen (zener / Geiger LFSR) ─► COLOR ─────────────────► MIX │         │
│                                                                   ▼         │
│                                                            ┌──────────┐    │
│   Bank Mode preset slider (vertical 5-pos) ─────────────────►  │ Tone     │    │
│   COLOR / WARM / DARK / COLOR / MIX                       │ Shaping  │    │
│                                                            └────┬─────┘    │
│                                                                 ▼          │
│                                                       OUT L (MAIN)         │
│                                                       OUT R (MAIN)         │
│                                                       DRY (separate)       │
│                                                       WET (separate)       │
│                                                                            │
│   GATE/CRUSH stomp ─► gate threshold + bitcrush sample-hold ─► insert      │
│                       в signal path post-mixer                             │
│                                                                            │
│   Feedback loop: wet → CARTRIDGE FEEDBACK pot → SW_FREEZE → driver         │
│                  (D_LIM soft clip, SPICE-stable)                           │
│                                                                            │
│   Solenoid damper: CV → Q5 → cartridge solenoid coil                       │
│                                                                            │
│   EG OUT: envelope follower output → external CV destination               │
│                                                                            │
└────────────────────────────────────────────────────────────────────────────┘

### Что происходит звуку (по блокам)

Stage 1 — input + dry split:

- Input buffer — high-Z (1МΩ для guitar compatibility), CMOS-switch true bypass relay (pedal version).

- Dry path split — для DRY OUT и dry mix bus.

Stage 2 — material resonance: 3. Pre-emphasis — shelving +8dB выше 3.2кГц для HF compensation материала. 4. Class AB driver — BD139/BD140 push-pull с bias diodes, R8 4.7Ω current limit, 3Вт peak на exciter. 5. EXCITER — Dayton DAEX25 (light) или DAEX32 (dense) на face A. 6. Physical plate — материал резонирует в bending modes. 7. Dual piezo pickup — Piezo A (bright, near exciter) + Piezo B (warm, far end). 8. JFET preamp LSK489A — dual matched, +27dB gain через mini-XLR shielded cables. 9. POSITION crossfade — A↔B mix (manual или CV). 10. De-emphasis — зеркало pre-emph.

Stage 3 — dual filter: 11. HiPass filter — variable, дает range от full-bass до telephone-band character. 12. LowPass filter — variable, controls top-end "veiled" feel.

Stage 4 — phaser sub-engine: 13. Phaser (Phase/Flutter) — 4-stage OTA-based all-pass network. RV_DEPTH controls phaser modulation depth; RV_SPEED controls LFO rate; SHAPE FORM slider — выбор LFO waveform (triangle / sine / random S&H / vinyl-skip pattern). 14. Phaser feedback — внутренний для resonant peaks.

Stage 5 — (vinyl FX block удалён в Decision 08 → переехал в Last Day как OLD VINYL PT2399 parallel tract):

- Vinyl-skip остаётся как одна из 5 позиций SHAPE FORM slider (LFO waveform), но больше не отдельный BBD pitch-warp block.

- Wow/flutter character доступен через phaser SPEED + Shape Form: vinyl-skip waveform для аналогичного эффекта.

Stage 6 — dynamics: 17. LED clipper — 3× LED in series each polarity, +10.6 dBu threshold. 18. Envelope follower VCA — LM13700, 1–48мс attack, 10мс–1с decay. Output = wet signal с dynamic shaping.

Stage 7 — mix + output: 19. Mix bus — DRY + WET + NOISE + CV mix. 20. Bank Mode slider — 5-position vertical: DIRTY / WARM / DARK / VOICE / MIX. DIRTY = noise color LPF + reverb dirt (moderate saturation). WARM/DARK/VOICE = reverb tone presets. MIX = combined moderate. 21. Outputs:

- MAIN L+R (stereo от dual piezo): A→L, B→R, или summed mono.

- DRY (отдельный jack): чистый сигнал post-buffer, без обработки.

- WET (отдельный jack): только обработанный сигнал, без dry.

- EG OUT: CV из envelope follower для патчинга в external modules.

Stage 8 — perform-FX (footswitch-driven): 22. GATE/CRUSH stomp: латчinг footswitch активирует gate (signal под threshold cuts) + bitcrush (sample-hold downsample) post-mixer. Постапокалипсис destruction effect. 23. FREEZE stomp: latches loop в self-sustain. 24. TAP stomp: tap-tempo для sync VINYL SPEED, phaser SPEED, или delay tail (Vinyl wow-rate sync к input rhythm). 25. BYPASS stomp: relay-buffered true bypass.

Параллельно:

- Noise generator — BZX55C9V1 zener (continuous hiss) или ATtiny LFSR (Geiger cluster pulses, premium).

- Cartridge feedback — wet signal → CARTRIDGE FEEDBACK knob → back to driver (pre-cartridge), создаёт self-oscillating drone на резонансе материала.

- Solenoid damper — CV/expression → mechanical plate mute.

### Важные параметры

| Параметр | Значение |
| --- | --- |
| Frequency response (без material, без FX) | 18Гц – 18кГц (-3dB) |
| Noise floor (input shorted, budget SKU) | -85 dBV |
| Noise floor (premium SKU, 4-layer PCB) | -95 dBV |
| THD (100мВ input, mid drive) | <0.5% |
| Phaser depth | 0–100% (4-stage all-pass) |
| Phaser SPEED range | 0.05–10 Гц |
| Noise generator architecture | Shared zener (BZX55C9V1) + ATtiny85 LFSR cluster ticks. NOISE knob = output level; COLOR(geiger) knob = continuous crossfader (CCW = hiss, CW = ticks) |
| Geiger tick rate | 0.5–20 events/sec @ random cluster pattern (ATtiny85 LFSR) |
| Solenoid double-function | DAMP sustained (CV) + TOLL impulse (5–10мс via J_TOLL_TRIG monostable) + STALL hold (J_STALL_CV forced) — CV-only triggers, no footswitch |
| Maximum input | +20 dBu (line level), +15 dBu typical |
| Maximum output | +12 dBu |
| Power draw (Eurorack) | ±12В, 250мА steady, 500мА peak |
| Power draw (Pedal) | 12V DC, 350мА typical / 500мА peak (включая phaser + cold palette + DC-DC losses) |
| Feedback loop stability | SPICE-verified для cartridges Q ≤ 1000 |
| Exciter peak power | 3Вт sustained, 8Вт peak |

## Контролы и интерфейс

Контролы идентичны для обеих SKU (одна схема). Раскладка отличается только enclosure-level — pedal vs Eurorack.

### Раскладка панели — Pedal SKU (~203×140мм, big-box class) — mockup canon

┌──────────────────────────────────────────────────────────────────────────────────┐
│  ◯                            LAST  NIGHT                                     ◯  │
│                            system suicide                                        │
│                                                                                  │
│  ◉ INPUT     ┌─[──────────────────────────────]─┐    ◉ DRY/WET   ◉ OUTPUT        │
│              │       CARTRIDGE SLOT             │                                │
│              │       horizontal — plate visible │    ◉ CARTRIDGE FEEDBACK        │
│              └─[──────────────────────────────]─┘                ▢ SWITCH CLIP   │
│                                                                                  │
│  ┌─COLOR─┐                                                                       │
│  │ COLOR │  ◉ DRIVE   ◉ ATK   ◉ DEC                                              │
│  │ WARM  │                                                                       │
│  │ DARK  │             ◉ LowPass            ◉ POSIT   ◉ BOOST                    │
│  │ COLOR │             ◉ HiPass                                                  │
│  │ MIX   │                                              ┌─── Shape Form ───┐    │
│  └───────┘                                              │  slider →         │    │
│                                                         │  ~ /\ ▲ ▼ ⌐⌐      │    │
│                                                         └───────────────────┘    │
│                                                                                  │
│  ◉ TONE   ◉ NOISE   ◉ COLOR(geiger)   ◉ Phase/Flutter   ◉ DEPTH   ◉ SPEED       │
│                       (crossfader)     (large)          (large)   (large)        │
│                                                                                  │
│  [Phase 2 v3 PCB upgrade adds: ◉ PULSE  ◉ FOG  ◉ FROST  ◉ CHILL  ◉ HUM ]        │
│                                                                                  │
├──────────────────────────────────────────────────────────────────────────────────┤
│  CV PATCH BAY (mini-jacks 3.5мм)                                                 │
│                                                                                  │
│  Row A: ⊙IN ⊙DRIVE ⊙DECAY ⊙NOISE ⊙POS ⊙DAMP ⊙LoPass ⊙MIX  ⊙EG(att) ⊙DRY ⊙WET    │
│  Row B: ⊙CLK ⊙TONE ⊙Attack ⊙COLOR ⊙FeedBack ⊙HiPass ⊙Boost ⊙TOLL ⊙STALL ⊙L MAIN R│
│                                                                                  │
├──────────────────────────────────────────────────────────────────────────────────┤
│  ▣ TAP        ▣ GATE-CRUSH                       ▣ BYPASS      ▣ FREEZE          │
│  (momentary)  (latching)                         (relay 3PDT)  (latching)        │
│                                                                                  │
│                  12V DC ⊕━○━⊖    EXP IN  ⊙                                       │
│  ◯                                                                           ◯   │
└──────────────────────────────────────────────────────────────────────────────────┘

Pedal layout особенности (203×140мм, big-box class — Strymon BigSky / Eventide H9 Max):

- 4 footswitches (mockup canon): TAP / GATE-CRUSH / BYPASS / FREEZE.

- 2 ряда CV patch bay (~22 mini-jack) — modular-style integration. Row B включает J_TOLL_TRIG и J_STALL_CV как modular-advanced triggers для solenoid double-function.

- Cartridge slot horizontal в top section с brackets на обоих концах — plate visible через окно 100×60мм.

- NOISE + COLOR(geiger) — 2 ручки (mockup canon): NOISE level + COLOR crossfader (continuous hiss ↔ Geiger ticks).

- Phase/Flutter, DEPTH, SPEED — large named knobs в lower row, always-on. Phaser равноправный эффект, не optional.

- Shape Form slider — horizontal 5-pos slider для LFO waveform select.

- Bank Mode preset slider vertical (5 positions) — quick tone preset selector.

- SWITCH CLIP toggle — soft/hard clipper mode.

- 12V DC jack (center-negative, 2.1мм barrel) + EXP pedal jack на rear.

- Phase 2 v3 PCB upgrade — добавляет 5 cold-palette knobs (PULSE/FOG/FROST/CHILL/HUM) в специально отведённом ряду.

### Раскладка панели — Eurorack SKU (40HP × 128.5мм)

Тот же набор контролов, разная компоновка:

┌──────────────────────────────────────────────────────────────────────────┐
│                            LAST  NIGHT                                   │
│                       system suicide                                     │
├──────────────────────────────────────────────────────────────────────────┤
│                                                                          │
│  ◉ INPUT  ◉ DRIVE  ◉ ATK  ◉ DEC  ◉ POSIT  ◉ BOOST  ◉ DRY/WET  ◉ OUTPUT  │
│                                                                          │
│  ┌─COLOR─┐  ◉ LowPass    ◉ HiPass         ┌── Shape Form slider ─────┐  │
│  │slider │                                 │                           │  │
│  │ COLOR │  ◉ TONE  ◉ NOISE  ◉ COLOR(geig) │                           │  │
│  │ WARM  │                                 │  ▢ SWITCH CLIP            │  │
│  │ DARK  │  ◉ Phase/Flutter  ◉ DEPTH      └───────────────────────────┘  │
│  │ COLOR │  ◉ SPEED   ◉ CARTRIDGE FB                                     │
│  │ MIX   │                                                               │
│  └───────┘                                                               │
│                                                                          │
│  ┌────────────────────────────────────────────────────────────────────┐ │
│  │           [─ C A R T R I D G E   S L O T ─]                        │ │
│  │      horizontal dock — magnetic mount + retention pin              │ │
│  │           100×60мм plate window между brackets                     │ │
│  └────────────────────────────────────────────────────────────────────┘ │
│                                                                          │
│  ▢ TAP btn   ▢ GATE-CRUSH btn   ▢ FREEZE toggle                          │
│  ◉ Activity LED  ◉ Clip L  ◉ Clip R  ◉ Damp LED                          │
│                                                                          │
├──────────────────────────────────────────────────────────────────────────┤
│  CV inputs (3.5мм Thonkiconn):                                           │
│  ⊙IN ⊙DRIVE ⊙DECAY ⊙NOISE ⊙POS ⊙DAMP ⊙LoPass ⊙HiPass ⊙MIX ⊙CLK          │
│  ⊙Attack ⊙TONE ⊙COLOR ⊙FB ⊙Boost ⊙TOLL ⊙STALL                            │
│                                                                          │
│  Audio:  ⊙ MAIN L  ⊙ MAIN R   ⊙ DRY OUT  ⊙ WET OUT  ⊙ EG OUT             │
└──────────────────────────────────────────────────────────────────────────┘

В Eurorack BYPASS не нужен (модуль просто отключается от patch). Footswitch-функции реализованы как buttons в нижней части панели + соответствующие gate inputs (для TAP, GATE-CRUSH, FREEZE).

### Полная таблица контролов

Phase 1 ship (ядро + базовые FX) — layout соответствует mockup:

| # | Контрол | Тип | Функция | CV input |
| --- | --- | --- | --- | --- |
| 1 | INPUT | Pot lin | Input gain (compatible с line level и instrument) | ✓ IN CV |
| 2 | CARTRIDGE FEEDBACK | Pot log | Уровень рециркуляции в plate-loop. На max → self-oscillation | ✓ FeedBack CV |
| 3 | DRY/WET | Pot log | Mix dry vs обработанного | ✓ MIX CV |
| 4 | OUTPUT | Pot lin | Master output level | — |
| 5 | DRIVE | Pot log | Уровень сигнала на exciter, глубина сатурации | ✓ DRIVE CV |
| 6 | ATK (Attack) | Pot log | Envelope follower attack 1–48мс | ✓ Attack CV |
| 7 | DEC (Decay) | Pot log | Envelope follower decay 10мс–1с | ✓ DECAY CV |
| 8 | POSIT (Position) | Pot dual-gang lin | Crossfade piezo A↔B (тембральный) | ✓ POS CV |
| 9 | BOOST | Pot lin | Pre-emphasis HF shelf 0 → +8dB | ✓ Boost CV |
| 10 | LowPass | Pot lin | LPF cutoff post-pickup | ✓ LoPass CV |
| 11 | HiPass | Pot lin | HPF cutoff post-pickup (dual-filter design) | ✓ HiPass CV |
| 12 | TONE | Pot lin | Master tone tilt EQ | ✓ TONE CV |
| 13 | NOISE | Pot log | Noise generator output level (shared zener + LFSR architecture, см. BUILD §Block 12) | ✓ NOISE CV |
| 14 | COLOR (geiger) | Pot lin | Crossfader noise character: CCW = continuous zener hiss → CW = Geiger cluster ticks (ATtiny85 LFSR). Параллельно работает LPF colour filter (white → brown). | ✓ COLOR CV |
| 15 | PHASE/FLUTTER | Pot log | Phaser feedback / resonance peak. Always-on named effect | — |
| 16 | rise slider | Vertical 30mm linear slider + exp converter | FG rise time (1ms–10s exponential) | ✓ rise CV |
| 17 | fall slider | Vertical 30mm linear slider + exp converter | FG fall time (1ms–10s exponential) | ✓ fall CV |
| 18 | depth slider | Vertical 30mm linear slider | FG output amplitude (0–100%, controls EG OUT and Iabc к phaser) | ✓ depth CV |
| 19 | exp/log | Pot lin center-detent | FG curve shape morph (exponential ↔ linear ↔ logarithmic) | — |
| 20 | speed/range | Pot log | FG rate (0.05–50 Hz exp) + clock sync continuous multiplier (CCW ×8 slow ↔ CW ×8 fast) | ✓ speed CV / CLK CV (TAP sync) |
| 21 | SW_FG_RANGE | 3-pos slide switch | FG range cap select (slow/mid/fast — swaps C_FG 1µF/100nF/10nF) | — |
| 22 | Bank Mode slider | Vertical 5-pos slider | DIRTY (noise color LPF + reverb dirt) / WARM / DARK / VOICE / MIX | — |
| 23 | SWITCH CLIP | 2-pos slide | Hard clip (LED) vs soft clip (diode) | — |

Phase 2 ship (cold-palette upgrade kit) — v3 PCB revision добавляет:

| # | Контрол | Тип | Функция | CV input |
| --- | --- | --- | --- | --- |
| 21 | PULSE | Pot lin | Slow LFO 0.05–2Гц на damper-pressure → tail "дышит" | ✓ PULSE CV |
| 22 | FOG | Pot lin | Apperiodic damper-mod (4066 S&H + slow slew) | ✓ FOG CV |
| 23 | FROST | Pot lin | HF absorber: VC-LPF в feedback loop sweep-down ВЧ | ✓ FROST CV |
| 24 | CHILL | Pot lin | Expander с brittle release (anti-compression) | ✓ CHILL CV |
| 25 | HUM | Pot lin | Ferrite-coil antenna pickup level (50/60Гц mains hum) | ✓ HUM CV |

### Footswitches (Pedal SKU — mockup canon)

| Switch | Тип | Функция |
| --- | --- | --- |
| TAP | momentary | Tap-tempo для phaser SPEED (sync через Shape Form vinyl-skip waveform для wow-feel). Sub-divisions через double-tap. |
| GATE/CRUSH | latching | Destruction effect post-mixer: gate (signal под threshold cuts) + bitcrush (sample-hold downsample). 4066 CMOS gate + LF398 crush cell. |
| BYPASS | relay 3PDT | Relay-buffered true bypass (no signal coloration when off). |
| FREEZE | latching | Feedback loop locks, контент бесконечно sustainит и деградирует. |

В Eurorack SKU те же четыре функции — большие кнопки на лицевой панели + соответствующие gate/CV inputs.

### TOLL / STALL — modular advanced features (CV-only triggers)

Solenoid double-function реализована на electrical уровне (один соленоид, два режима — sustained DAMP + impulse TOLL strike), но доступна через CV mini-jacks, не через footswitches:

| CV input | Тип | Функция |
| --- | --- | --- |
| J_TOLL_TRIG | gate (5V trigger) | Короткий impulse → solenoid strike → bell-tone пластины. Через NE555 monostable 5–10мс pulse + OR-gate с DAMP path. |
| J_STALL_CV | sustained CV (+5V hold) | Forced full damper hold. Длительный high → decay stuck at minimum. Release = decay resumes. |

Modular users patch sequencer gates → TOLL для rhythmic bell-strikes. Pedalboard users могут использовать через expression-jack adapter (CV-trigger box) — но это advanced workflow.

Внутри electrical implementation идентична Decision 08 — solenoid double-function через 555 monostable + OR-gate diodes. Изменена только entry-point (CV jacks вместо footswitches).

⚠ STALL safety: STALL CV — momentary gesture, max 5 секунд continuous high при default setup. Beyond that — firmware PWM throttle automatically dims coil drive к 40% duty (sufficient hold force) для thermal protection. См. BUILD §Block 14 thermal budget.

### CV Patch Bay (Pedal: ~21 jacks; Eurorack: ~15 jacks)

Все controls имеют CV destination. Список:

Audio I/O:

- IN (audio input, 1× jack)

- MAIN L, MAIN R (stereo output)

- DRY OUT (separate dry signal post-buffer)

- WET OUT (separate wet signal без dry)

CV inputs (Phase 1 — main row 1 of CV bay):

- IN CV (input level)

- gate CV (Block 18 gate threshold)

- attack CV (envelope follower attack)

- decay CV (envelope follower decay)

- drive CV

- tone CV

- lopass CV

- feedback CV (cartridge feedback)

- position CV (Position crossfade)

- boost CV (pre-emphasis shelf amount)

- speed CV (FG rate / clock sync)

CV inputs (main row 2):

- clock CV (TAP/CLK in для FG sync)

- noise CV (noise generator level)

- color CV (geiger crossfader: hiss ↔ ticks)

- output CV (master output level)

- mix CV (dry/wet blend)

- HiPass CV (HPF cutoff)

- rise CV (FG rise time mod)

- fall CV (FG fall time mod)

- depth CV (FG depth mod)

Modular advanced (extras zone, frame-grouped):

- J_TOLL_TRIG — solenoid strike trigger. Internally normalled от FG EOR (each FG peak → bell strike). Patch external = breaks norm.

- J_STALL_CV — sustained CV for forced damper full hold (decay stuck minimum while high; max 5s continuous per thermal budget).

- J_SIDE — sidechain input для envelope follower (normalled к main input; patch external для dub-style ducking).

- J_FREEZE_CV — gate input для freeze. OR-logic с footswitch state — либо footswitch latched, либо CV > 0.7V → freeze active. Use cases: rhythmic stutter freezes, sequencer-driven freeze patterns.

CV inputs (Phase 2 upgrade adds):

- PULSE CV, FOG CV, FROST CV, CHILL CV, HUM CV

Outputs (CV):

- EG OUT (main row 2) — FG main waveform output, scaled by depth slider. 0–5V envelope-style.

- Gate_OUT (extras zone) — FG rise-active gate (HIGH during rise phase, LOW during fall).

- Sub÷2_OUT (extras zone) — FG ÷2 rate sub-output (flip-flop divider).

- Inv_OUT (extras zone) — FG inverted main waveform.

Pedal-only:

- EXP IN (expression pedal jack, normalled к user-assignable target)

Audio outputs (back panel jacks):

- MAIN L / MAIN R (stereo TRS + RCA parallel)

- DRY OUT / WET OUT (separate post-buffer taps)

### Cartridge connector (внутри slot, идентичен в обеих SKU)

| Connector | Покрытие | Функция |
| --- | --- | --- |
| TA3M (mini-XLR) ×2 | Shielded coax 50мм | Piezo A, Piezo B (low-noise audio) |
| JST-XH 2-pin ×2 | Twisted pair | Exciter drive, Solenoid coil |

## Картриджи — каталог материалов

### Формат картриджа

- Внешний размер рамки: 110×65×30мм (PETG 3D-print prototype, фрезерованный алюминий production).

- Plate dimensions: длина 100мм fixed, высота 20–60мм per material, толщина 0.5–6мм per material.

- Mount: 4× neodym N42 магниты align + spring-loaded retention pin.

- Connectors: 2× mini-XLR (Piezo A, B) + 2× JST-XH (Exciter, Solenoid).

- Mass: ~50г frame + plate-зависимая масса.

### Phase 1 catalog — 6 cartridges (initial launch)

| Картридж | Размеры (Д×В×Т, мм) | Масса plate | RT60 (без feedback) | Exciter | Pickup | Retail |
| --- | --- | --- | --- | --- | --- | --- |
| OAK RAW | 100×40×4 | 12г | 0.1–0.3с | DAEX25FHE-4 | 2× piezo 27мм | $80 |
| OAK LINSEED | 100×40×4 | 12г | 0.15–0.35с | DAEX25FHE-4 | 2× piezo 27мм | $90 |
| MAPLE SHELLAC | 100×35×4 | 10г | 0.2–0.5с | DAEX25FHE-4 | 2× piezo 27мм | $110 |
| MARBLE | 100×50×5 | 68г | 0.8–2с | DAEX32Q-4 | 2× piezo 27мм | $150 |
| BRASS | 100×30×3 | 77г | 1–3с | DAEX32Q-4 | 2× piezo 27мм | $160 |
| SPRING STEEL | 100×20×0.5 | 8г | 2–6с | DAEX25FHE-4 | 2× piezo 27мм | $130 |

### Phase 2 additions (после ship Phase 1)

| Картридж | Размеры | RT60 | Характер | Retail |
| --- | --- | --- | --- | --- |
| EBONY | 100×25×3 | 0.3–0.8с | Премиум, длинный sustain, тёмный | $180 |
| NEPHRITE — Sayan green | 100×40×4 | 1–3с | Поющий, медитативный, bell-like, 4000-летняя традиция (bianqing 编磬). ~48г. DAEX32 exciter. См. cartridges/07_nephrite_processing.md | $250 |
| NEPHRITE — Hetian "mutton fat" white | 100×40×4 | 1.5–4с | Премиум limited 50 экз., белый "баранний жир" — visual contrast в каталоге. Mirror-polished с diamond paste | $450 |
| NEPHRITE — pounamu (NZ) | 100×40×4 | 1–3с | Ritual edition limited 25 экз., Ngāi Tahu treaty-certified. Глубокий "kahurangi" зелёный | $550 |
| COPPER | 100×40×1.5 | 1–2.5с | Колокольный, patina меняет тембр | $170 |
| PYREX GLASS | 100×40×3 | 1–3с | Crystalline chime (compatible с Be Careful) | $140 |
| BONE (lopatka) | 100×40×4 | 0.3–0.8с | Мрачный, ритуальный, sourced ethically. См. cartridges/05_bone_processing.md | $190 |
| TITANIUM | 100×30×1.5 | 3–8с | Ultra-premium, max Q, infinite ring | $350 |

### Premium tier cartridges (с PVDF pickup)

Premium SKU использует PVDF film на B-канале вместо piezo disk → нейтральная АЧХ, position crossfade становится тембральным control "coloured ↔ neutral" (не просто "ближе/дальше").

| Картридж | Tier | Premium up-charge |
| --- | --- | --- |
| Standard line (oak, marble, brass...) | Budget | — |
| Standard line (premium SKU) | Premium | +$30 (PVDF on B-channel) |

### Sound character (per material, в более detail)

#### Wood — теплота органическая

- Oak raw: матовый, mid-focused, как звук удара по dry wood. Короткий decay. Идеален для perкуссии и vocals.

- Oak linseed: чуть ярче чем raw (закрытые поры), more sustained. Готов для studio reference.

- Maple shellac: скрипичный, музыкальный, harmonic richness. Шеллак — самое музыкальное покрытие. Premium woody.

#### Stone — пространство монументальное

- Marble: cathedral. Длинный пространственный tail, dense, тёплый. Для drone и ambient.

- Slate (Phase 2): тёмный, comb filter character.

#### Metal — shimmer и резонанс

- Brass: bell-like, ситарный buzz, восточный character. Singing bowl в reverb form.

- Spring steel: classic plate-like shimmer, infinite ring при high feedback. Самый "обычный reverb" звук.

- Copper (Phase 2): mellow, warm metal, patina aging добавляет character.

- Titanium (Phase 2): glassy, чистый, max Q. Premium endgame cartridge.

#### Glass — кристаллическая хрупкость

- Pyrex (Phase 2): высокий Q narrow band-pass character. Compatible с Be Careful filter module — universal cartridge platform.

#### Bone & exotic

- Bone scapula (Phase 2): сухой, ритуальный, matt sustain. Source: cattle (food industry byproduct, ethical).

- Mammoth ivory (limited edition): между bone и stone. $400–600 retail. Limited 50 units.

### Cartridge swap procedure

- Power down module (или mute через FREEZE during swap).

- Pull spring-loaded retention pin (small lever).

- Slide cartridge out (magnetic detach).

- Slide new cartridge in until magnets engage.

- Push retention pin back.

- Done. Ready to play.

Hot-swap при включенном питании — возможен, но возможен short pop при mating. Recommend mute или freeze before swap.

### Care и maintenance

- Wood cartridges: re-oil каждые 6–12 месяцев (1 drop linseed oil, polish с lint-free cloth).

- Glass cartridges: handle с care, не drop. Storage в padded foam box.

- Metal cartridges: чистить от oxidation если patina нежеланна. Brass tarnishes naturally.

- Bone cartridges: keep dry. Не expose к humidity (warping risk).

- Stone cartridges: heaviest, careful с rack mounting (don't drop into rack).

## Тембральный характер

### Общий character

Постапокалиптичный. Звук пустоты, не зала. Каждый удар оставляет физический отпечаток на материале — слышно, как пластина "помнит" сигнал. На high feedback — drone "коридора, по которому никто не идёт".

### По регистру

- Низ (50–200Гц): зависит от материала. Дерево скрашивает, мрамор резонирует, тонкий металл не передаёт.

- Середина (200Гц–2кГц): главный sweet spot модуля. Все materials говорят здесь.

- Верх (2кГц–10кГц): зависит от exciter и pre-emphasis. Дерево заваливает, стекло подчёркивает.

- Воздух (10кГц+): minimal без помощи pre-emphasis.

### По динамике

Envelope follower VCA reagирует на input dynamics:

- Громкие удары → wet signal в полную силу → выраженный reverb tail.

- Тихие ноты → wet signal ослаблен → почти dry с micro-tail.

- Sustained tones → envelope settles → constant wet level.

- Percussion → быстрый attack envelope → dynamic shaping.

Это даёт естественную dynamic compression — не нужен external sidechain для transparent reverb.

### Saturation behavior

При high DRIVE сигнал перегружает class AB driver → BD139/BD140 saturate → adds even-harmonic distortion. Не транзисторный clipping — softer, ламповый character. Combined с material resonance — даёт "warm overdrive с tail".

### Feedback character

- 0–30%: subtle reinforcement, slight tail extension.

- 30–60%: noticeable reverb с cleanly extended decay.

- 60–80%: long evolving tail, slight modulation от feedback dynamics.

- 80–95%: edge of self-oscillation, dramatic sustained drone.

- 95–100%: self-oscillating на dominant material resonance. Frequency depends on cartridge material.

D_LIM1/D_LIM2 soft-clip diodes prevent runaway amplitude (verified SPICE), но не frequency stability — self-oscillation на резонансной частоте материала возможна и intended.

### Position crossfade character

- Full A position: bright, immediate, "близкий" звук — ear-on-the-plate.

- Center: mixed both pickups — fullest sound.

- Full B position: warm, diffuse, "далёкий" звук — ear-в-room.

С PVDF B-pickup (premium) — A→B sweep становится "coloured analog → neutral hi-fi" sweep. Tonal control, не spatial.

### Noise generator character

- White noise (COLOR full CW): full-spectrum hiss, "tape hiss" feel.

- Brown noise (COLOR full CCW): low-frequency rumble, "wind in empty room".

- Mid color: pink-ish, broadband but warm.

Phase 2 v2 (optional Geiger pattern firmware): impulsive clicks instead of continuous hiss → "post-apocalyptic radiation count".

## Сценарии использования

### Patch 1 — Vocal reverb (studio)

Vocal mic → preamp → Last Night IN
Cartridge: OAK SHELLAC (musical, не overpowering)
DRIVE: 9 o'clock (clean)
FEEDBACK: 11 o'clock (mid tail)
MIX: 1 o'clock (heavy wet)
TONE: 2 o'clock (slight HF roll-off)
POSITION: 12 (center)
ATTACK: 9 (fast)
DECAY: 1 (medium hold)

OUT → DAW interface

Result: warm vocal reverb с natural dynamics, не overpowering.

### Patch 2 — Synth lead drone (Eurorack)

VCO → VCF → Last Night IN
Cartridge: SPRING STEEL (long shimmer)
DRIVE: 12 (mid drive)
FEEDBACK: 3 o'clock (long tail)
FREEZE: ON (locked drone)
MIX: full wet
ATTACK: full clockwise (slow swell)
DECAY: full clockwise (max sustain)

CV from Fuck Abandoned Sleep pendulum LFO → CV_DAMP
(slow modulation of solenoid damper → breathing drone)

OUT → mixer

Result: 30-секундный sustained drone с pendulum breathing.

### Patch 3 — Percussion (live)

Drum machine → Last Night IN
Cartridge: BRASS (bell-like)
DRIVE: 3 o'clock (saturated)
FEEDBACK: 9 (short)
MIX: 11 (mostly dry с slight wet)
TONE: 12 (neutral)
POSITION: 9 (bright A)
ATTACK: 9 (immediate)
DECAY: 11 (short)

Performance: kick triggers via SIDE → percussive bell-tail.

OUT → live mixer

Result: "brass bell" перkussive accent over drums.

### Patch 4 — Ambient pad (compositional)

Pad synth → Last Night IN
Cartridge: MARBLE (cathedral)
DRIVE: 11 (gentle)
FEEDBACK: 1 (long)
MIX: 3 (heavy wet)
TONE: 1 (warm)
POSITION: 2 (warm B)
ATTACK: 12 (medium)
DECAY: 3 (long)

External LFO → CV_MIX (slow swell of wet level over 60 секунд)

OUT → headphones / monitors

Result: cathedral ambient pad с slow evolution.

### Patch 5 — Noise / experimental (industrial)

Noise source → All Bones Dust → Last Night IN
Cartridge: SPRING STEEL
DRIVE: full CW (over-driven)
FEEDBACK: 4 o'clock (near self-osc)
MIX: full wet
TONE: 3 (bright)
NOISE: 3 (heavy noise mix)
COLOR: 12 (white noise)

External gate → CV_DAMP (rhythmic damping)

OUT (L) → Last Day IN (для feedback bridge)
Last Day OUT → BACK to Last Night IN (через mixer)
= Infinite Day↔Night feedback loop

Result: industrial noise wash с rhythmic clamping.

### Patch 6 — Day → Night transition (full system)

Synth voice → All Bones Dust → Be Careful → IS MY → Last Day → AND MY → Last Night → OUT

AND MY balance: slowly automated 0 → 100% over 4 minutes
(starts at Last Day mostly, ends at Last Night fully)

= Composition arc от raskалённого полдня в холодную ночь

Result: 4-минутная композиция, проходящая через всю серию.

### Patch 7 — Guitar pedal (live)

Guitar → Last Night IN (Hi-Z compatible с R1=1МΩ)
Cartridge: MAPLE SHELLAC (musical)
DRIVE: 12 (clean)
FEEDBACK: 11 (short tail)
MIX: 12 (50/50)
TONE: 1 (warm)

Footswitch (FREEZE momentary) — hold for sustain freeze
Expression pedal → MIX (foot-controlled wet/dry)

OUT → guitar amp

Result: traditional reverb pedal с swappable acoustic character.

### Patch 8 — DJ FX send/return (live performance)

DJ console FX SEND → Last Night IN (RCA если premium SKU)
Cartridge: MARBLE (cathedral)
DRIVE: 1 (heavy)
FEEDBACK: 3 (long)
MIX: full wet (post-FX bus)

Master beat → SIDE input (sidechain pumping)

OUT → console FX RETURN (RCA)

Result: rhythmic pumping reverb wash, dub-style.

## Технические характеристики

### Electrical

| Параметр | Значение |
| --- | --- |
| Power supply | ±12В Eurorack bus (modular) или 12V DC center-negative + TRACO TMR 3-1212WI / Recom RKD-1212-D isolated DC-DC (pedal) |
| Power consumption | 200мА steady, 450мА peak (с solenoid pulse) |
| Audio input | 1× 3.5мм TS, Hi-Z (1МΩ), max +20 dBu |
| Sidechain input | 1× 3.5мм TS, line level |
| Audio outputs | 2× 3.5мм TS (mono mix L, stereo R via normalling) |
| CV inputs | 3× 3.5мм TS (mix, decay, damp), 0–10В range |
| Frequency response (without cartridge) | 18Гц – 18кГц (-3dB) |
| Output impedance | 1кΩ |
| Maximum output | +12 dBu before clipping |
| THD (100мВ in, mid drive) | <0.5% |
| SNR (input shorted) | 85 dB (budget), 95 dB (premium 4-layer PCB) |
| Crosstalk L↔R | <-60 dB @ 1кГц |

### Mechanical

| Параметр | Значение |
| --- | --- |
| Form factor | Eurorack 3U × 40HP (203.2мм × 128.5мм panel) |
| PCB | 2-layer FR4 1.6мм (budget) / 4-layer FR4 1.6мм (premium) |
| PCB dimensions | 101.6 × 108мм |
| Depth (behind panel) | 35мм (excluding cartridge dock) |
| Cartridge dock depth | 30мм additional |
| Total module depth | 65мм |
| Weight (module без cartridge) | 220г |
| Weight (typical cartridge) | 60–130г |
| Mounting | M3 screws, 4 corners |

### Connectors

| Connector | Specification |
| --- | --- |
| Audio in/out | Thonkiconn PJ301M-12 (3.5мм TS, vertical) |
| CV in | Thonkiconn PJ301M-12 |
| Cartridge piezo | 2× Switchcraft TA3M (mini-XLR, locking, shielded) |
| Cartridge exciter | JST-XH 2-pin |
| Cartridge solenoid | JST-XH 2-pin |
| Power | 2×5 IDC (Eurorack standard) или DC jack 12V (pedal) |

### Cartridge interface

| Параметр | Значение |
| --- | --- |
| Mount | 4× neodymium N42 magnets + spring-loaded retention pin |
| Hold force | ~5N (resistant к casual rack handling, releases с deliberate pull) |
| Magnetic field at piezo trace | <0.1мТ (negligible audio interference) |
| Insertion cycles | >1000 specified |
| Hot-swap | Possible (с FREEZE engaged), но не recommended (audible pop on mate) |

### Cartridge plate physics (typical wood)

| Параметр | Значение |
| --- | --- |
| Plate dimensions | 100мм × 20–60мм × 0.5–6мм (variable) |
| Mode density | ~950 modes below 10кГц для oak 100×40×4мм |
| Fundamental frequency | 200–400Гц typical for wood |
| RT60 unforced | 0.1–0.3с (oak) до 6с (spring steel) |
| Q factor | 50 (oak) до 1000+ (titanium) |

### Compliance

| Standard | Status |
| --- | --- |
| CE marking | Compliant (low-voltage directive, EMC) — full certification по Phase 2 |
| RoHS | Compliant |
| REACH | Compliant |
| FCC (US) | TBD по Phase 2 (для US market entry) |
| WEEE | Compliant (EU recycling) |

### Warranty

| Тип | Срок |
| --- | --- |
| Parts & labor | 1 год |
| Cartridges | 30 дней (defect только, не accidental damage) |
| Glass cartridges | Excluded (хрупкость as designed) |
| Out-of-warranty repair | Flat fee $50 + parts |
| Lifetime | Replacement components доступны 5 лет (2N7000, BD139/140, общедоступные); LSK489A критическая часть, обеспечена через dual-source strategy |

## Совместимость и patching

### Eurorack совместимость

- Power: ±12В bus (стандарт), 2×5 IDC. Reverse polarity protection через 1N5817.

- HP: 40HP — flagship size. Fits в 84HP, 104HP, 168HP racks (occupies нижнюю половину). Same size as Last Day (зеркальная пара).

- Depth clearance: 65мм total — fits в стандартные case (Tiptop Mantis 60мм может не fit; Doepfer А-100 шкафы — yes).

### Pedal совместимость

- Power: 12V DC center-negative, 2.1мм barrel jack, 500mA min, regulated. Это modern standard для complex pedals (Strymon TimeLine $449, Eventide H9 $499, Meris LVX $599, Chase Bliss CXM 1978 — все 12V). 9V supplies не подходят — нужны 12V outputs от pedalboard PSU. Compatible: Voodoo Lab Pedal Power 4×4, Cioks DC7/DC10, Eventide PowerMax, Strymon Zuma R300, MXR M238 ISO-Brick (на 18V port — внимание polarity).

- Bypass: relay-bypassed buffered. Selectable true bypass через internal jumper.

- TRS jacks: main I/O 6.3мм TS, CV inputs 3.5мм mini-jack patch bay.

- Footswitches: TAP (momentary), GATE/CRUSH (latching), BYPASS (relay), FREEZE (latching).

### Compatible с другими SYSTEM SUICIDE модулями

- Fed by: I Show You Light, Body Blood And Salt, All Bones Dust, Be Careful, Last Day.

- Modulated by: Fuck Abandoned Sleep (CV from pendulum LFO), Is My (gates/clocks), And My (balance crossfade с Last Day).

- Feeds: And My (input в crossfader), final OUTPUT.

### Compatible с external Eurorack modules

- VCO: any standard Eurorack VCO (Make Noise, Mutable, Erica Synths).

- VCF: any filter — Last Night sits после filter.

- Sequencer: trigger в SIDE input или CV в DAMP for rhythmic effects.

- LFO: external LFO в любой CV input для modulation.

- Mixer: standard summing mixer для multi-source patches.

### Compatible с external pedal chains

- Position в chain: typically last в reverb/delay slot. Fits хорошо после fuzz/distortion, before delay/looper.

- Send/return loops: works в FX loops с -10dBV или +4dBu levels.

- DJ console: RCA option (premium) for FX send/return integration.

### Patching tips

- Avoid overload: high DRIVE + bright cartridge + high BOOST → может перегрузить downstream. Watch CLIP LEDs.

- Self-oscillation: high FEEDBACK на high-Q cartridges (titanium, spring steel) → controlled drone. Tune frequency через POSITION crossfade.

- Side-chain pumping: kick через SIDE input → pumping reverb wash. Adjust DECAY to taste.

- Stereo from mono: A→L, B→R через position crossfade + stereo OUT — pseudo-stereo effect from single source.

- Infinite loop: с Last Day — connect Last Day FB SEND → Last Night SIDE, Last Night OUT → Last Day FB RETURN. Bidirectional feedback.

## Уход и обслуживание

### Регулярный уход

- Cartridge swap: до 1000 cycles specified. После 500+ swaps — visual inspection магнитов на debris.

- Pots: используются Alpha RV09 quality. Контактная очистка DeoxIT каждые 2 года при heavy use.

- Cartridge dock cleanliness: blow dust от mini-XLR connectors каждые 6 месяцев compressed air.

- PCB inspection: при weird noise — open chassis, visually inspect для cold solder joints, capacitor bulging.

### Wood cartridge re-finishing

Через 6–12 месяцев wood cartridges могут потерять surface finish:

- Linseed oil cartridges: drop линс oil на surface, polish с lint-free cloth, 24h cure.

- Shellac cartridges: одно coat shellac alcohol-thinned (1:3), brush, 12h cure, light sanding (#400), polish.

- Raw wood cartridges: just dust off. Optional: light wax for protection.

### Glass cartridge handling

- NEVER drop — glass cartridges are fragile.

- Storage: foam-padded box или dedicated cartridge wallet.

- Transport: never в bare rack. Always padded.

- Если cracked — discard, не try to play (oscillation amplification может shatter).

### Metal cartridge maintenance

- Brass: tarnishes naturally. Clean с brass polish (Brasso) для shine, или leave для patina character.

- Spring steel: rust prevention — light oil coat (3-in-1 oil) каждые 12 месяцев. Wipe excess.

- Copper: develops green patina, может change tone subtly. Feature, not bug.

- Titanium: maintenance-free. Doesn't oxidize.

### Stone cartridge handling

- Heavy. Mount carefully в rack — drop risk.

- Marble может chip if struck — handle с two hands.

### Bone cartridge

- Keep dry — humidity warps.

- Don't expose к direct sunlight (UV damage).

### Module repair

Common issues:

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| No sound output | Power issue, blown fuse | Check ±12V на ICs, verify 1N5817 not shorted |
| Hum 50/60Гц | Ground loop | Check star-ground tie at power conn, replace cable |
| Clicks при solenoid activation | EMI coupling в piezo | Verify mini-XLR cables shielded, ferrite bead на solenoid |
| Self-oscillation runaway | Feedback loop instability | Check D_LIM diodes installed correctly, test SPICE if recurring |
| One channel dead | JFET preamp failure | Test LSK489A с DMM, check bias resistors |
| Solenoid не activates | MOSFET gate issue | Verify R_DAM1 = 47кΩ, gate voltage at CV 5В should be ~3.4В |
| Wood cartridge pitch sounding "thin" | Plate cracked or detached от exciter | Replace cartridge |
| Distortion на line level | LED clipper threshold low | Verify D1/D2 = 3× LED in series (not single LED) |
| Hiss above noise floor | Input source / cable / preamp issue | Test с input shorted — should be -85dBV или quieter |

Repair cost (out of warranty):

- Component replacement: $30 flat fee + parts cost.

- Major rework (PCB damage): $80 + parts.

- Cartridge replacement: $80–250 depending material.

### Software/firmware updates (если v2 noise generator с MCU)

Если future version имеет ATtiny85 для Geiger-pattern noise:

- USB-DFU update через bootloader (no programmer needed).

- Update file посылается customer email при release.

- Backwards compatible: original analog mode остаётся accessible.

### End of life

При финальном retirement модуля:

- Module: WEEE compliant disposal (Europe), e-waste (US).

- Cartridges: reusable, sell к other Last Night owners.

- Custom cartridges (laser-etched ones): collector items.

## Контакты и поддержка

- Manual download: ardet.systems/manual/last-night-v2.pdf

- Cartridge catalog: ardet.systems/cartridges

- Email support: support@ardet.systems

- GitHub (open source schematic, closed PCB): github.com/chervycola/ardet/synth

Last Night v2.2

"I show you light, blood, bones, dust. Be Careful. Fuck Abandoned Sleep. Is my Last Day And my Last Night."