# Nephrite (Jade) cartridge processing — от raw slab до полированной пластины

"Stone that sings" — нефрит звучал в китайских ритуальных оркестрах за 2000 лет до нашей эры (bianqing 编磬). Один из самых вязких природных материалов с исключительной акустической добротностью.

Полный workflow обработки сырого нефритового slab'а для cartridge plate 100×40×4мм.

Total elapsed time: 1–2 weeks (большая часть — lapidary processing, не passive waiting like bone). Active labor: ~6–8 часов на cartridge. BOM cost: ~$25–40 per finished plate (зависит от source nephrite quality). Retail tier: $250 (Phase 2 premium cartridge SKU).

## Содержание

- Почему нефрит

- Sourcing — где взять

- Cutting — резка пластины

- Coarse grinding — выравнивание thickness

- Sanding progression — шлифовка

- Polishing — финальная полировка

- Mounting в cartridge frame

- Acoustic optimization

- Safety — техника безопасности

- Variants — стилистические варианты

- Cost & timeline

## 1. Почему нефрит

### Физические свойства

Nephrite — calcium-magnesium silicate с химической формулой Ca₂(Mg,Fe)₅Si₈O₂₂(OH)₂. Одно из двух минералов, которые традиционно называют jade (второе — jadeite, ещё реже и дороже).

Ключевые параметры:

- Плотность: 2.9–3.1 g/cm³ (тяжелее древесины, легче мрамора, плотнее костяной).

- Твёрдость Mohs: 6–6.5 (умеренно твёрдый — требует diamond tools, но не такой жёсткий как кварц или сапфир).

- Toughness: исключительная — нефрит один из самых вязких природных материалов, благодаря interlocking fibrous microstructure. Не раскалывается легко.

- Acoustic Q: очень высокий (300–800 для cortical-grade material) — длинный sustain, чистый ringing tone.

- Цвет: типичен green (от железа), также white ("mutton fat"), yellow, brown, black ("Black Jade").

### Историческое использование как acoustic material

Древний Китай (>2000 лет до н.э.) — нефрит использовался для bianqing (编磬) — наборы L-образных нефритовых пластин, подвешенных в раме. Удар деревянным молотком давал чистый bell-tone, sustainoм в минуты. Каждая пластина настроена на свою ноту (полная гамма).

Это самый ранний задокументированный пример использования материальной пластины как настраиваемого музыкального резонатора. Last Night cartridge buys в эту 4000-летнюю традицию.

Современный пример: Maori pounamu chimes — новозеландский нефрит, ритуальный mauri stone. Pounamu карвинги звучат при ношении на шее.

### Тембральный character (для cartridge use)

- Долгий sustain: 1–4 секунды unforced (vs oak 0.1–0.3с, marble 0.8–2с, spring steel 2–6с).

- Singing, медитативный — не "cathedral" как marble, не "infinite shimmer" как steel. Cредневековый клиросный характер.

- Bell-like overtones — гармонический спектр с выраженными 2-й, 3-й, 5-й гармониками.

- Высокий Q означает risk feedback self-oscillation при aggressive cartridge feedback knob. Это feature, не bug — drone potential.

### Acoustic comparison к другим cartridges

| Material | Density | Q-factor | RT60 unforced | Character |
| --- | --- | --- | --- | --- |
| Oak (raw) | 0.75 g/cm³ | 50–80 | 0.1–0.3с | Тёплый, перkussive |
| Maple (shellac) | 0.7 g/cm³ | 100 | 0.2–0.5с | Скрипичный |
| Nephrite (Сибирский) | 3.0 g/cm³ | 300–500 | 1–3с | Поющий, медитативный |
| Marble | 2.7 g/cm³ | 300 | 0.8–2с | Cathedral |
| Spring steel | 7.8 g/cm³ | 800 | 2–6с | Long shimmer |
| Titanium 1.5мм | 4.5 g/cm³ | 1000+ | 3–8с | Glass-like |

Нефрит — уникальный middle ground между tonal warmth (древесина) и sustained резонансом (металл). Отсутствует в большинстве commercial reverb продуктов — uniqueness factor для Last Night.

### Почему именно nephrite (не jadeite)

- Nephrite — более доступный, дешевле, более устойчивый к cracking. Подходит для cartridge production.

- Jadeite — рарер, harder (Mohs 6.5–7), дороже (premium gem grade $1000+ per slab). Не повседневный cartridge material — если использовать, то limited edition только.

## 2. Sourcing — где взять

### Option A — Sayan / Siberian nephrite (рекомендуется для проекта)

Сибирский / саянский нефрит — основной российский источник, исторический материал для post-Soviet rock и lapidary crafts.

- Месторождения: Кавоктинское, Буромское (Бурятия), Оспинское (Иркутская обл.).

- Цвет: преимущественно зелёный с тёмными пятнами, иногда yellow-green.

- Качество: cortical grade (cartridge-suitable) commonly $30–80/kg в rough slab form.

- Доступность: онлайн-магазины камня (Россия, Казахстан), gem shows.

Russian suppliers:

- Lapidary shops в Москве, СПб, Иркутске.

- Online: yandex поиск "сибирский нефрит сырьё", "rough nephrite", "нефрит кабошон сырьё".

- Цена: $30–80/kg для cartridge-grade rough.

### Option B — Hetian nephrite (Chinese premium)

Хотанский нефрит — historical premium, исторически связан с Шёлковым путём. Самый дорогой grade.

- Месторождения: Hetian (新疆和田) в Синьцзяне.

- Цвет: prized "mutton fat white" (баранный жир), также green, yellow.

- Цена: $200–1000/kg для cartridge-grade material.

- Доступность: китайские online suppliers (Alibaba), специализированные jade shops.

Use case: Premium limited edition cartridge (white "mutton fat" — visual contrast к остальному каталогу). $450 retail tier.

### Option C — New Zealand pounamu

Маорийский нефрит: ritualnoe значение. Pounamu sales регулируются законом (Ngāi Tahu treaty). Покупать только legal source с certificate.

- Цвет: depth green, "kahurangi" (translucent), "tangiwai" (dark).

- Цена: $150–500/kg.

- Доступность: ограничена законом, требует verification.

Use case: artistic statement cartridge, "World Materials Edition". Не для регулярного каталога.

### Option D — Wyoming / BC Canadian jade

- Месторождения: Wyoming (US), British Columbia (Canada).

- Цвет: variable green-grey-black.

- Цена: $50–200/kg.

- Доступность: lapidary supply chains North America.

### Какой выбрать для Last Night Phase 2 catalog

Recommend: Option A — Siberian / Sayan nephrite.

Обоснование:

- Цена: $30–80/kg — feasible для retail $250 cartridge.

- Доступность: local supply chain в России и Восточной Европе.

- Quality: cortical-grade nephrite даёт excellent acoustic properties.

- Цвет: зелёный — visual contrast к древесным cartridges каталога.

- History: 100+ лет использования в советских/российских ювелирных traditions.

Phase 2 premium SKU: Option B — Hetian "mutton fat" white как limited edition (50 экземпляров max), retail $450+.

### Что просить у lapidary supplier

При заказе:

- "Cortical-grade nephrite slab" — без significant cracks или inclusions.

- Размер raw slab: minimum 120×60×8мм (запас под cutting waste).

- Если возможно — pre-cut к 110×50×6мм (cost +20%, экономит твоё время на rough cutting).

- Cвидетельство provenance (для documentation cartridge serial).

## 3. Cutting — резка пластины

### Tools

Главный: wet diamond tile saw (та же, что для bone cartridge — см. 05_bone_processing.md секция 7).

- Diamond blade: 4" (100мм) wet saw blade, continuous-rim sintered diamond. Стоимость: $30–60.

- Water reservoir обязателен — diamond сухой режим перегревается и tooth wear катастрофический.

Альтернативы (не рекомендуются для production, OK для prototype):

- Angle grinder с diamond disc — менее точно, больше dust, риск injury.

- Lapidary trim saw — точнее, но дороже ($300+).

### Cutting procedure

- Plan cuts на slab — pencil markings или dry-erase marker show outline (110×50×6мм рабочий размер — запас 10% на dry cutting + sanding).

- Secure slab в clamp. Nephrite toxic не является, но dust раздражает дыхание — wet cutting only.

- Wet tile saw setup: water reservoir full, blade clean.

- Slow feed: ~5–8 мм/sec (медленнее чем для bone, нефрит твёрже).

- Cooling pause каждые 30 секунд — blade и stone не перегреваются.

- Кromоk полировка: после rough cut кромки можно слегка skруглить diamond burr или wet sand 100-grit (предотвращает chipping в дальнейшем).

### Yield

Из slab 200×120×10мм можно вырезать 2–4 пластины 110×50×6мм с минимальными отходами.

### Cutting safety

- N95+ mask + face shield — silicate dust от нефрита аналогичен silicosis-risk при chronic exposure.

- Только wet cutting.

- Eye protection mandatory — stone chips летят при saw kickback.

- Hearing protection — diamond saw громкий.

## 4. Coarse grinding — выравнивание thickness

После cutting пластина rough — толщина варьирует ±0.5мм, поверхность с blade marks. Coarse grinding выравнивает thickness к 4.5мм (немного выше target 4мм, оставляя material для sanding).

### Tools

Diamond cup wheel на bench grinder или lapidary flat lap:

- 4" (100мм) diamond cup wheel, 80–120 grit. $30–50.

- Mounted на standard 1/2" arbor bench grinder, или dedicated flat lap.

Альтернативы:

- Belt sander с silicon carbide belt 60-grit — slower, work, но cheaper.

- Hand sanding с 60-grit SiC — exhausting, только для single prototypes.

### Grinding procedure

- Mark target thickness carefully на edge каждой стороны (4.5мм reference).

- Slow grinding strokes, full plate contact с cup wheel.

- Water cooling continuous — stream от bottle или dedicated water supply.

- Rotate plate каждые 30 секунд — даёт even wear.

- Check thickness каждые 1 минуту с digital caliper — nephrite removed faster than expected.

- Цель: 4.5мм ±0.1мм uniform thickness, без visible saw marks.

### Time estimate

- Per plate coarse grinding: 15–25 минут active work.

### Common mistakes

- Over-grinding: nephrite не gives "warning" перед перешлифовкой — становится тоньше plan'а quickly. Calipers check often.

- Unever pressure: одно edge тоньше другого. Rotate plate frequently.

- Heat cracking: dry grinding или insufficient water → thermal stress → micro-cracks (visible или invisible). Wet always.

## 5. Sanding progression — шлифовка

Goal: убрать grinding marks, выровнять surface, prepare for polish. Многоступенчатая progression от coarse к fine grit.

### Grit progression

| Stage | Grit | Purpose | Tool |
| --- | --- | --- | --- |
| 1 | 100-grit SiC | Remove coarse cup wheel marks | Wet sanding на belt sander или flat surface |
| 2 | 220-grit SiC | Smooth, начать flatness | Hand or belt |
| 3 | 400-grit diamond | Fine smoothing | Hand sanding, wet |
| 4 | 600-grit diamond | Near-polish prep | Hand, wet |
| 5 | 1200-grit diamond | Mirror prep | Hand, wet |
| 6 | 3000-grit diamond | Pre-polish | Hand, wet |

Critical: diamond grits после 400 — silicon carbide too soft для эффективной работы с нефритом >400-grit equivalent.

### Sanding procedure (per stage)

- Apply sandpaper на flat backing block (3D-printed PETG или granite slab).

- Wet sand — keep water continuous на surface (dry sanding heats stone, можно crack).

- Cross-pattern strokes: 30 секунд one direction, 30 секунд perpendicular. Prevents linear scratches.

- Check progress каждый stage — должно быть видно prior-stage scratches исчезают.

- Clean stone тщательно между stages — leftover grit от previous stage scratches.

### Final check после 3000-grit

- Surface должна иметь slight sheen, "satin" finish.

- Held against light — slight reflection visible.

- Surface не должна иметь visible scratches при naked eye inspection.

### Time estimate

- Per plate, all stages: 1.5–2 часа active work.

## 6. Polishing — финальная полировка

Nephrite polishes до glass-like mirror finish при правильной обработке. Этот этап критически важен — определяет visual quality + acoustic clarity (микро-roughness увеличивает damping).

### Polishing compounds для нефрита

| Compound | Particle size | Stage | Carrier |
| --- | --- | --- | --- |
| Cerium oxide | 0.5–1 μm | Final mirror polish | Water slurry |
| Tin oxide | 1–3 μm | Pre-mirror, традиционный | Water |
| Chromium oxide | 0.3–0.5 μm | High-gloss alternative | Wax stick |
| Diamond paste 8000–14000 grit | 0.5–1 μm | Premium, fastest | Oil-based |

Recommend: Cerium oxide для production (cheap, predictable, традиционный для jade).

### Polishing procedure

#### Method A — Felt wheel + cerium oxide slurry (стандарт)

- Mount felt wheel на bench polisher или drill press chuck. 4" (100мм) felt wheel — $10.

- Mix slurry: 1 столовая ложка cerium oxide + 3 ст ложки воды → paste consistency. Apply на wheel surface.

- Apply plate к wheel при low speed (1000–1500 RPM), light pressure.

- Keep slurry wet — добавлять water через spray bottle continuously.

- Polish 5–10 минут per plate — visible glass-like sheen develops.

- Inspect под bright light: surface должна reflect like glass. Если remain dull spots — repeat polish.

- Final wash: detergent + warm water → убирает residual slurry. Towel dry.

#### Method B — Hand polish (DIY, slower)

Для prototype или low-volume:

- Cerium oxide paste на felt pad (без power tool).

- Rub firmly в circular pattern, 15–30 минут per plate.

- Same final result, но more elbow grease.

#### Method C — Diamond paste premium

Для premium SKU:

- Diamond paste 8000 + 14000 grit sequential.

- Leather strop для final mirror.

- Result: deeper mirror, less surface haze. Premium "mutton fat" white nephrite particularly benefits.

### Mirror quality check

После finalpolish:

- Light reflection test: hold near LED lamp, surface reflects sharp source.

- Fingernail test: гладкая на ощупь, без any "rough" spots.

- No scratches visible с naked eye.

### Stabilization (optional)

Unlike bone (требует beeswax/shellac protection), polished nephrite не нуждается в stabilization — material itself stable, не hygroscopic, не absorbing oils от skin contact significantly.

Optional: mineral oil polish (1 drop, rub с soft cloth) — slight depth enhancement, food-safe.

### Time estimate

- Per plate polishing: 30–45 минут (Method A or C).

- Hand polish (Method B): 60–90 минут.

## 7. Mounting в cartridge frame

Final processed plate (4мм thick, 100×40мм, mirror polished) монтируется в cartridge frame аналогично другим материалам:

### Procedure

- Inspect frame (PETG print или milled aluminum, 110×65×30мм).

- Apply rubber rails в top/bottom slots: 2× silicone strips 2мм thick, adhesive backing.

- Insert plate sliding sideways в frame. Compression от rubber rails holds it.

- Mount exciter к face A: Dayton DAEX32Q-4 (heavy duty — nephrite at 48g borderline между DAEX25 light и DAEX32 dense range). M3 bolt через rubber gasket uncoupler.

- Glue piezo discs на face B:

  - Piezo A: ~33мм from one edge (near exciter mounting position).

  - Piezo B: ~67мм from same edge (far position).

  - Cyanoacrylate gel (CA glue), 24h cure.

- Solder cables:

  - Piezo A coax 50мм → TA3F mini-XLR (Pin 2 signal, Pin 1 shield).

  - Piezo B coax 50мм → second TA3F.

  - Exciter twisted pair 50мм → JST-XH 2-pin.

  - Solenoid wiring → second JST-XH (cartridge level solenoid).

- Test electrical: DMM check piezo каждый канал (~10нФ capacitance), exciter (~4Ω DC), solenoid (~5–10Ω).

- Laser-etch serial на frame edge: SYSTEM SUICIDE / LAST NIGHT / NEPHRITE / SAYAN / SN: 0001.

- Pack в foam-lined box с material card.

### Exciter choice — DAEX25 или DAEX32?

- Nephrite plate 100×40×4мм: масса ~48г.

- DAEX25FHE-4 spec: до 30г material weight.

- DAEX32Q-4 spec: до 60г.

Recommend: DAEX32Q-4 для nephrite cartridges. Higher power handling, better coupling к heavier material. BOM impact: $40 vs $20 (premium tier).

## 8. Acoustic optimization

### Mass check

Cortical Sayan nephrite density: 2.95–3.05 g/cm³. For 100×40×4мм plate: mass ≈ 47–49г.

- <40г: material too porous или wrong stone type. Reject.

- 47–49г: правильный density, expected acoustic character.

- >52г: unusually dense (premium Hetian grade?). Excellent для premium SKU.

### Tap test

После full processing, hold plate by edge в clamp или fingers, tap с pencil eraser по центру:

- Sharp "ping" с sustain 1–3 секунды — отлично, premium cartridge.

- "Pong" с sustain 0.5–1с — acceptable, standard tier.

- Dull "thunk", short sustain — likely micro-cracks от cutting/grinding. Reject или re-process от raw slab.

- "Ring" с sustain >3с unforced — exceptional, mark for premium tier.

### Exciter mounting position

Nephrite grain isotropic (interlocking fibrous microstructure без preferred direction). Поэтому exciter mounting:

- Off-center, ~33мм from one edge.

- Avoid corners (high-stress points).

- Mounting через rubber uncoupler — preserve plate Q.

### Piezo positions

- Piezo A (bright): 33мм from same edge as exciter — captures direct excitation.

- Piezo B (warm): 67мм from same edge — captures decay tail.

- Position crossfade gives smooth bright↔warm gradient.

### Expected acoustic spec

For Siberian Sayan nephrite plate 100×40×4мм:

- RT60 unforced: 1–3 seconds.

- Character: singing, медитативный, harmonic-rich.

- Q-factor: 300–500.

- Self-oscillation threshold: low — high feedback легко даёт drone. Documented as feature.

### Feedback loop interaction

Nephrite high-Q + Last Night feedback loop → very easy to push to self-oscillation. Players love this — "drone cartridge".

SPICE verification: при cartridge Q=500, feedback loop gain margin снижается vs oak (Q=50). Loop stable до ~70% feedback knob position. Above — controlled self-oscillation на dominant material resonance frequency (typically 800Гц–1.5кГц для 100×40×4мм nephrite).

## 9. Safety — техника безопасности

### Health hazards

| Stage | Hazard | СИЗ |
| --- | --- | --- |
| Raw slab handling | Sharp edges от saw cuts | Cut-resistant gloves |
| Wet cutting | Stone aerosol, water splash | N95 mask, face shield, apron |
| Dry grinding (AVOID) | Silicate dust — silicosis-like при chronic exposure | P100 respirator MANDATORY, vacuum collector |
| Wet grinding | Reduced aerosol, water containment | N95 mask, eye protection |
| Sanding | Stone dust + sandpaper grit | N95 mask, vacuum |
| Polishing | Cerium oxide dust (minor irritant) | N95 mask, ventilation |

### Critical "NEVER"

- NEVER cut/grind/sand DRY — silicate dust extremely hazardous при chronic exposure. Wet only.

- NEVER use non-diamond tools для grinding/sanding >220 grit — silicon carbide rapidly wears out, не effective против Mohs 6+ material.

- NEVER apply heat к нефритovoy пластине (например, hot air gun для bonding) — thermal stress может create invisible micro-cracks.

- NEVER mix wet processing с electrical tools без GFCI — water + electricity = electrocution risk.

### Environment

- Wet cutting/sanding station: covered drainage, water settling tank (cerium oxide и stone dust settle out для disposal).

- Stone slurry disposal: dry the slurry, dispose в regular trash. Don't pour wet slurry в sink (settles, clogs).

- Used polishing rags: keep wet или dispose properly — dry cerium oxide easily becomes airborne.

### Ethical considerations

- Pounamu (NZ jade): legal restrictions, требует certificate of provenance, supports Ngāi Tahu treaty.

- Hetian (Chinese): source verification важна — некоторые operations use abusive labor. Check supplier ethics.

- Sayan (Russian): typically OK, established mining cooperatives.

## 10. Variants — стилистические варианты

### Standard tier — Siberian green

Сибирский зелёный нефрит, polish to satin sheen, mineral oil treatment. $250 retail. Default Phase 2 catalog entry.

### Premium — Hetian "mutton fat" white

Хотанский белый нефрит, premium SKU. Visual rarity (most cartridges зелёные/тёмные — белый stands out). Mirror polish с diamond paste. $450 retail. Limited edition 50 units max.

### Ritual edition — pounamu (New Zealand)

Маори jade с legal provenance certificate. Premium spiritual SKU. $550 retail. Limited 25 units, requires Ngāi Tahu treaty verification.

### Carved — relief pattern

After polishing, hand-carve system suicide motif (small relief 5–10мм). Filled с ink или left raw. Premium $600+ retail — labor intensive.

### Black jade

Russian dark jade variety — rare colour. Premium aesthetic. $350 retail.

### Banded multi-color

Natural banding (зелёный + белый + brown layers). Visually distinctive. $300–400 retail depending на pattern quality.

## 11. Cost & timeline

### Per cartridge (DIY single)

| Этап | Активное время | Общее время | Стоимость |
| --- | --- | --- | --- |
| Source raw nephrite (1.5–2 kg slab) | 30 мин (order) | 3–7 дней (delivery) | $50–120 |
| Cutting к 110×50×6мм | 30 мин | — | $5 (blade wear) |
| Coarse grinding к 4.5мм | 20 мин | — | $2 (cup wheel wear) |
| Sanding progression (6 stages) | 90 мин | — | $3 (diamond paper) |
| Polishing (cerium oxide) | 40 мин | — | $1 (polish compound) |
| Mounting в cartridge frame | 30 мин | — | $30 (frame + exciter + piezos + cables + packaging) |
| Acoustic test + serial | 15 мин | — | $1 (laser etch) |
| ВСЕГО | ~4.5 часа | 1 week elapsed | ~$92–162 |

Из 1.5–2кг slab — обычно 2–4 cartridges при правильном planning, что снижает per-cartridge cost.

### Серийная партия (10 cartridges, batch)

- Заказать 10 large slabs (или 3–4 super-slabs дающих 3–4 plates каждый): $400–600.

- Cutting + grinding + sanding в batch на shared equipment: 2 рабочих дня.

- Polishing: один день.

- Mounting + testing: один день.

- Total active labor (10 cartridges): ~20–25 часов.

- Per-cartridge cost in batch: ~$60–80.

### Розничный tier

- Standard Siberian nephrite BOM: $90–110.

- Retail: $250.

- Gross margin: ~55–65%.

- Premium Hetian BOM: $200–300.

- Retail: $450–550.

- Gross margin: ~50%.

Менее aggressive margin чем bone ($11 BOM → $190 retail = 94% margin), но более высокая retail price tier = $25–40 absolute profit per cartridge vs $18 для bone.

### Производительность

Solo workshop с wet tile saw + bench polisher + lapidary flat lap:

- 3–5 nephrite cartridges per week (active mode).

- 120–200 cartridges per year.

Достаточно для small premium boutique demand. Limited edition tiers (Hetian, pounamu) — ниже capacity, выше margin.

## Workshop checklist (для prototype)

- ☐ Order Sayan nephrite slab 200×120×10мм from lapidary supplier (2 weeks lead time).

- ☐ Set up wet tile saw с water reservoir.

- ☐ Get bench grinder + 4" diamond cup wheel (100-grit).

- ☐ Bench polisher или drill press + felt wheel 4".

- ☐ Diamond sandpaper set: 400 / 600 / 1200 / 3000 grit (wet/dry).

- ☐ Silicon carbide paper 100 / 220 grit (for early stages).

- ☐ Cerium oxide polishing compound (50г bag, $15).

- ☐ DAEX32Q-4 exciter (for nephrite mass class).

- ☐ PPE: N95 masks, P100 respirator (for any dry contingency), nitrile gloves, eye protection.

- ☐ Digital calipers ($20).

- ☐ Granite flat reference stone (для flatness verification).

- ☐ Cartridge frame с rubber rails ready.

## Notes

- First nephrite cartridge будет slower than estimate — lapidary skill ramp. Expect 6–8 hours active first attempt. By 3rd cartridge — efficient 4-hour workflow.

- Stone quality varies significantly within single slab — sometimes side has inclusions, other side clean. Inspect post-cutting, before grinding committing к thinning.

- Document each plate с photo + acoustic measurement — track quality variance batch к batch.

- Slow и steady wins — nephrite Mohs 6.5 не tolerates rushed processing. Each stage with proper time.

- Premium positioning: nephrite cartridge — это luxury cartridge в каталоге. Marketing emphasize 4000-year history (bianqing, pounamu), не just material.

Камень, который поёт.