# Decision 10 — Elite tier components & sourcing strategy

Дата: 2026-05 Статус: draft v1 (open для refinement after first prototype sourcing) Тип: SKU tier definition + supply chain strategy

## Контекст

Last Night имеет три canonical tier:

| Tier | BOM per pedal | Retail | Customer |
| --- | --- | --- | --- |
| Budget | ~$94 | $499 | Standard boutique pedal buyer |
| Premium | ~$121 | $649 | Audiophile prosumer (Strymon BigSky tier) |
| Elite | $300–600 (TBD this doc) | $1,499–2,499 | Boutique collector / Eventide H9000 / Strymon big-box audiophile |

Elite SKU — это art-object SKU, limited edition с высоким absolute margin ($600+ per unit), наглядной premium ощутимостью (heavy brass corpus, exotic wood inlays, audiophile-grade signal path), provenance documentation для каждого экземпляра.

Этот документ:

- Каталог premium/elite component options по каждому блоку схемы.

- Sourcing matrix — где их доставать.

- Total Elite SKU BOM расчёт + retail tier proposal.

- Risk register: discontinued / hard-to-find / long lead-time компоненты.

## 1. Op-amps (TL072 / TL074 baseline)

Текущие baseline ICs — workhorse TL07x ($0.50–0.75). Не аудиофильский tier.

| Tier | Part | $ | Использовано | Notes |
| --- | --- | --- | --- | --- |
| Budget | TL072CP / TL074CN | $0.50–0.75 | U1, U2, U3, U4, etc. | Standard, JFET input, OK для most stages |
| Premium | OPA1652 / OPA1654 | $3 / $5 | All audio path stages | TI, ultra-low noise (4.5 nV/√Hz), bipolar input, drop-in TL07x replacement |
| Premium alt | NE5532 (BJT) | $1 | Bus buffers | Classic, slightly more noise чем OPA1652 |
| Elite | OPA627AU / OPA1612 | $35 / $8 | Critical preamp / output stages | OPA627: legendary audiophile op-amp. OPA1612: modern equivalent, lower noise |
| Ultra-elite | Sparkos Labs SS3601 (discrete op-amp module) | $75 each | Output stage только | Replacement module, drop-in SOIC-8. 4 нV/√Hz, audiophile prestige |
| Boutique | Burson V6 Vivid discrete op-amp module | $90 each | Output stage | Premium discrete, audio-jewellery tier |

Recommendation для Elite SKU:

- Replace U1 (input buffer) + U2 (mix/output) с OPA1612 (cheaper modern audiophile, ~$8 each) = $32 BOM.

- Output stage exclusive: Sparkos Labs SS3601 ×1 ($75) = signature audiophile sound.

- All filter / EQ stages: OPA1652 ($3 each × 4 = $12).

- Tier subtotal: ~$130 just для op-amp upgrade.

## 2. JFETs (LSK489A baseline)

Уже выбран premium LSK489A для piezo preamp. Дальнейший upgrade путь:

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget legacy | 2N5457 (EOL) | $1 | Discontinued, no production option |
| Premium baseline (current) | LSK489A | $6 | Linear Integrated Systems, dual matched in SOT-23-6 |
| Elite | LSK489B / LSK170A | $10 | LIS premium grade с tighter Idss matching ±5% vs ±15% |
| Ultra-elite | 2SK170 BL grade (NOS Toshiba) | $30+ | Legendary Japanese JFET, discontinued 2007, NOS only. Pairs hand-matched by ear |
| Boutique | Linear Systems LSK489 selected | $15 | LIS factory-selected for low noise (<1 nV/√Hz) |

Recommendation Elite: LSK489B selected ($10). Noise floor advantage = +3dB SNR над baseline LSK489A. Significant для piezo signal which is наша Achilles heel.

## 3. OTAs (LM13700 baseline)

LM13700 — workhorse. Premium ОТА для VCA + phaser:

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget (current) | LM13700N | $2 | Texas Instruments, classic OTA, character distortion ~0.5% |
| Premium | THAT2180A | $8 | Professional audio VCA, distortion <0.05%. Used в SSL, API, etc. |
| Premium quad | SSI2164 | $15 (quad) | Sound Semiconductor, modern SSM2164 replacement. 4× VCAs в одном чипе, Eurorack-popular |
| Elite | CEM3387 (NOS) | $80 (rare) | Curtis chip, vintage Prophet-5 character. Discontinued ~1990s |
| Boutique | Sound Semiconductor SSI2161 | $10 | Modern single VCA premium, low noise |

Recommendation Elite:

- U5 (Block 11 envelope VCA) → THAT2180A ($8) для tighter VCA control.

- U6, U7, U8 (Block 12 crossfader + Block 16 phaser) — оставить LM13700 (character важна для phaser sweep).

- BOM delta: +$6 над baseline.

## 4. Capacitors

Самая аудиофильская категория с самой огромной price spread.

### Signal path coupling caps (C_IN, C_OUT, C_CA, C_CB):

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget | Generic film box | $0.15 | Function only |
| Premium baseline | WIMA MKS2 1µF film | $0.30 | German, audio standard, low ESR |
| Premium audio | WIMA MKP4 polypropylene | $0.80 | Better dielectric, less microphonic |
| Elite | Auricap XO 1µF | $8 each | Audiophile polypropylene, hand-selected. Used в high-end DAC outputs |
| Boutique | Mundorf Supreme Silver/Gold/Oil 1µF | $35–60 | Top tier audiophile cap, silver foil + oil-paper |
| Ultra-boutique | Audio Note KAISEI 1µF | $80+ | Japanese audiophile cult cap, copper foil + paper-in-oil |

### EQ caps (C_PE1, C_DE1 — 1nF C0G):

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Premium baseline | Murata GRM1885C1H102JA01D 1nF C0G ±5% | $0.05 | Already specified |
| Elite | Cornell Dubilier silver mica 1nF | $3 | Vintage radio character, very low loss |
| Boutique | Vishay/Roederstein MKT 1nF film | $2 | Audiophile film, tight tolerance |

### Bulk electrolytics (C_DC, C_B1, C_B2):

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Premium baseline | Panasonic EEU-FR series 1000µF | $0.30 | OK |
| Premium audio | Nichicon Muse FW 1000µF | $2 | Audio-rated electrolytic, lower ESR |
| Elite | Elna Silmic II 1000µF | $5 | Japanese audiophile silmic series, silk-fibre electrolyte. Famous "warm" character |
| Boutique | Black Gate (NOS, hyper-rare) | $50+ | Legendary discontinued Japanese audiophile cap. Cult status |

Recommendation Elite SKU:

- C_IN, C_OUT (2× signal path): Mundorf Supreme 1µF ($40 × 2 = $80).

- C_PE1, C_DE1 (matched pair EQ): Cornell Dubilier silver mica ($3 × 2 = $6).

- C_DC (1000µF bulk): Elna Silmic II ($5).

- Decoupling 100nF × 10: WIMA FKP2 film ($0.40 × 10 = $4) instead of ceramic.

- Subtotal: ~$95 BOM upgrade only for caps. Significant.

## 5. Resistors

Стандартные 1% metal film = OK для audio. Elite tier:

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget | Generic 1% metal film | $0.01 | Function only |
| Premium | Dale CMF55 | $0.10 | RN-style precision, low TCR, low noise |
| Premium audio | Vishay/Beyschlag MBA0207 | $0.30 | Audio-grade, ultra-low noise |
| Elite | Audio Note tantalum non-magnetic | $5+ each | Japanese audiophile resistor cult |
| Boutique | Shinkoh tantalum (discontinued NOS) | $20+ each | Legendary, paper-tape mounting |
| Engineering elite | Vishay VAR / Z-foil precision | $5–15 | Tightest tolerance ±0.005%, lowest TCR. Used в reference voltage sources |

Recommendation Elite SKU:

- Signal path resistors (R_in, R_out, summing, EQ network): Dale CMF55 ($0.10 × ~30 = $3).

- Critical preamp gain resistors (R_GA, R_RA в Block 9 JFET preamp): Audio Note tantalum ($5 × 4 = $20).

- Все остальные: standard 1% metal film.

- Subtotal: ~$25 BOM upgrade.

## 6. Pots & sliders

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget | Alpha RV09 9mm | $1.20 | Workhorse, OK |
| Premium baseline | Alpha RV09 sealed | $1.80 | Dust-sealed for live use |
| Premium audio | Alps RK09L | $3.50 | Japanese, smoother feel, longer life |
| Premium pro | Alps RK27 "Blue Velvet" | $25 | Audiophile cult — used в Marantz, Yamaha hi-fi. 27mm large pot |
| Elite | TKD CP-2500 | $50–80 | Tokyo Ko-On Denpa, Japanese boutique audio pot |
| Boutique stepped attenuator | Khozmo 24-step | $80+ | Discrete resistor stepped attenuator, precision control |
| Ultra-boutique | Goldpoint precision stepped | $200+ | American audiophile high-end |

### Sliders (Color preset 4P5T + Shape Form 1P5T):

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget | Generic 4P5T slide switch | $5 | OK, может ratty контакты |
| Premium | Alpha SL-4P5T | $8 | Cleaner contacts |
| Premium pro | NKK SS-12 series | $15 | Japanese pro slide switch |
| Elite | C&K 7000 series | $25 | Mil-spec gold-plated contacts |

Recommendation Elite SKU:

- Большие knob'ы performance gestures (DRIVE, FEEDBACK, MIX, OUTPUT, DEPTH, SPEED): Alps RK27 Blue Velvet ($25 × 6 = $150).

- Mid-tier knobs (ATK, DEC, POSIT, BOOST, TONE, NOISE, COLOR-geiger, LowPass, HiPass): Alps RK09L ($3.50 × 9 = $32).

- Color preset slider 4P5T: C&K mil-spec ($25).

- Shape Form slider 1P5T: NKK ($15).

- Subtotal: ~$222 BOM upgrade.

## 7. Switches & footswitches

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget | Generic 3PDT footswitch | $3 | Often Chinese clones, contact issues |
| Premium | Cliff Electronics 3PDT | $7 | UK-made, reliable contacts |
| Premium pro | Carling Technologies 3PDT | $12 | American mil-spec, gold-plated |
| Elite | Carling 110 series sealed | $18 | Sealed against dust + moisture |
| Boutique | Otax soft-touch | $25 | Japanese soft-click silent footswitch |

### SWITCH CLIP toggle:

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget | C&K 7101 generic | $1.50 | OK |
| Premium | Honeywell 1MD1-T1 | $5 | Mil-spec |
| Elite | Schurter MS-series | $12 | Swiss premium with embedded LED option |

Recommendation Elite SKU:

- 4 footswitches: Carling 110 sealed ($18 × 4 = $72).

- SWITCH CLIP: Schurter MS ($12).

- Subtotal: ~$84 BOM upgrade.

## 8. Jacks (audio + CV)

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget 6.3mm | Generic TRS jack | $1 | Often poor contact |
| Premium | Switchcraft 11/12 series | $3 | American workhorse, audio standard |
| Premium audio | Neutrik NMJ4HCD-S | $5 | Gold contacts, robust |
| Elite | Neutrik NJ3FP6C | $8 | Locking TRS, gold contacts |
| Boutique | WBT-0210 nextgen | $40 | Silver-plated nextgen, audiophile cult |

### CV mini-jacks:

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget | Thonkiconn PJ301M-12 | $1 | Eurorack standard |
| Premium | Thonkiconn gold-plated | $2 | Same form, gold contacts |
| Elite | Switchcraft N141-style | $4 | American premium 3.5mm |

Recommendation Elite SKU:

- 6.3mm main I/O × 4: Neutrik gold ($5 × 4 = $20).

- RCA stereo × 2: Switchcraft RCA gold ($3 × 2 = $6).

- 22 CV mini-jacks: Thonkiconn gold ($2 × 22 = $44).

- DC power jack 2.1mm barrel: Switchcraft 712A ($2).

- Subtotal: ~$72 BOM upgrade.

## 9. Cartridge cable connectors

Уже chose Switchcraft TA3M/TA3F mini-XLR (premium baseline). Elite alternatives:

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Premium baseline | Switchcraft TA3M/F mini-XLR | $3 each | Solid baseline |
| Premium pro | Neutrik miniCON | $5 | European premium, locking |
| Elite | Lemo PIB.0M (3-pin push-pull) | $25 | Swiss precision, military aerospace grade |
| Boutique | Hirose HR10 series | $15 | Japanese audiophile/medical grade |

Recommendation Elite SKU:

- TA3M/F → Lemo PIB ($25 × 2 pairs = $100).

- Subtotal: $96 BOM upgrade (replaces baseline $12).

## 10. Cables (cartridge internal coax)

Baseline: generic RG-174 50мм за $1.

| Tier | Part | $/m | Notes |
| --- | --- | --- | --- |
| Budget | Generic RG-174 | $1/m | OK function |
| Premium | Belden 8451 (instrument cable) | $2/m | Studio standard |
| Premium audio | Mogami W2549 | $4/m | Japanese audiophile internal cable |
| Elite | Canare L-4E5C | $6/m | Mogami competitor, star-quad construction |
| Boutique | Furutech FA-220 | $20/m | Furutech audiophile internal |

Recommendation Elite SKU:

- 4× cartridge internal coax 50мм: Mogami W2549 ($4/m × 0.2m × 4 = $3.20).

- Subtotal: ~$2 BOM upgrade (cheap relatively to other components).

## 11. Exciter

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget (light cartridges) | Dayton DAEX25FHE-4 | $20 | Workhorse, 4Ω |
| Premium (dense cartridges) | Dayton DAEX32Q-4 | $40 | More power handling |
| Premium German | Visaton EX 60 S | $50 | German engineering, audiophile drivers |
| Premium driver | Tectonic BMR4 (Balanced Mode Radiator) | $35 | Different excitation modes |
| Elite | Custom-wound voice coil exciter | $150+ | Commission через Tang Band или specialty exciter shops |
| Boutique | Mass Loaded Vinyl (MLV) hybrid exciter | $200+ | Audiophile cult — heavier voice coil mass |

Recommendation Elite SKU:

- Visaton EX 60 S ($50) — proven, German precision, modest upgrade.

- For limited-edition Hetian nephrite cartridges — custom-wound premium exciter (commission).

- Subtotal: $30 BOM upgrade.

## 12. Piezo / pickup

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget | Generic 27mm piezo disc | $0.50 | OK, mid-resonance peak |
| Premium | Murata 7BB-27-4L0 premium disc | $2 | Better consistency |
| Premium audio | PVDF film LDT0-028K (Measurement Specialties) | $5 | Flat frequency response, no mid peak |
| Elite | K&K Pure Mini pickup | $40 | Cult acoustic guitar piezo, bright/clear |
| Boutique | Schertler DYN-V piezo | $80 | Swiss audiophile transducer |
| Ultra-boutique | Schertler STAT-V condenser piezo | $250 | Studio-grade |

Recommendation Elite SKU:

- Piezo A (bright): K&K Pure Mini ($40).

- Piezo B (warm): PVDF film ($5) — different timbre per pickup creates dramatic Position crossfade character.

- Subtotal: $45 BOM upgrade over $1 baseline.

## 13. PCB

| Tier | Specs | $ per pcs (small batch) | Notes |
| --- | --- | --- | --- |
| Budget | 2-layer FR4 1.6mm HASL | $5–10 (JLCPCB / PCBWay) | Function OK |
| Premium | 4-layer FR4 1.6mm ENIG (gold) | $30–60 (JLCPCB) | Impedance controlled, gold plating |
| Elite | 6-layer with internal ground/power planes, immersion silver | $80–150 (JLCPCB premium) | Audio-grade routing, lower noise |
| Boutique | Custom Rogers RO4350 high-frequency substrate | $200–400 | Reference-grade RF/audio |
| Ultra-boutique | Hand-routed point-to-point на ebonite chassis | $500+ (artisan) | Cult tube amp builders' approach |

Recommendation Elite SKU:

- 6-layer FR4 ENIG ($100) — best balance audio quality + manufacturability.

- Subtotal: $90 BOM upgrade.

## 14. Enclosure

| Tier | Material | $ | Notes |
| --- | --- | --- | --- |
| Budget | Stamped aluminum (Hammond-like) | $15 | Functional |
| Premium | Anodized milled aluminum 6063-T5 | $40 | Decent feel, common pedalboard |
| Premium pro | CNC-milled 6061-T6 aerospace aluminum | $80 | Pro feel, premium tooling marks |
| Elite | Solid brass corpus | $250 | Heavy (~2кг pedal!), beautiful patina развивается |
| Elite alt | Solid copper corpus | $300 | Will develop green patina — matches "postapocalypse rust" aesthetic perfectly |
| Boutique | Walnut / ebony wood-clad sides + brass top | $400+ | Artisan craftsmanship, art-object tier |
| Ultra-boutique | Cast bronze sculpture-shaped enclosure | $1000+ | One-of-one art piece |

Recommendation Elite SKU:

- Copper corpus ($300) — patinates green с возрастом, matches "system suicide" aesthetic (mёртвая electronics, oxidized metal). Visually unique against most aluminum pedals.

- Alternative: solid brass ($250) — heavier, gold-like patina.

- Subtotal: $260 BOM upgrade over $40 premium baseline.

## 15. Knobs

| Tier | Part | $ each | Notes |
| --- | --- | --- | --- |
| Budget | Generic Davies clones | $0.50 | OK |
| Premium baseline | Davies 1900H (Selco authentic) | $3 | Classic synth knob, made в Japan |
| Premium pro | Re'an AS-200/AS-300 | $4 | Boutique aluminum knobs |
| Premium audiophile | Soulution / Vicor brass knobs | $15 | German audiophile machined brass |
| Elite | Custom-molded Bakelite | $25 | Period-correct postapocalypse aesthetic, real Bakelite (not plastic clone) |
| Boutique | Antique radio knobs (NOS 1930-1950s) | $20–80 | Real vintage, varied availability |
| Ultra-boutique | Solid brass machined knobs custom | $40+ each | Custom commission |

Recommendation Elite SKU:

- 20 knobs total на panel (including Phase 2 v3 PCB upgrade).

- Mix: 8 large performance knobs brass machined ($40 × 8 = 320),12smaller**Bakelitemolded**(25 × 12 = $300).

- Альтернатива: full Bakelite ($25 × 20 = $500) — more cohesive aesthetic.

- Subtotal: ~$500 BOM upgrade. Самая большая отдельная категория.

## 16. DC-DC (pedal SKU power)

| Tier | Part | $ | Notes |
| --- | --- | --- | --- |
| Budget | TRACO TMR 3-1212WI | $13 | 3W isolated, current spec |
| Premium baseline | Recom RKD-1212-D | $22 | 250mA per rail, audio-rated |
| Premium audio | Recom REC15-1212S with EMI shielding | $35 | Shielded, lower noise |
| Premium | Mean Well DKE15B-12 | $30 | Premium audio-grade DC-DC |
| Elite | Linear regulated PSU (no switching) | $50 | Heavier, no switching noise, audiophile cult |
| Boutique | External audiophile linear PSU (e.g., Cardas / Mojo Audio) | $200+ | External brick supply, completely isolated audio chain |

Recommendation Elite SKU:

- Recom REC15-1212S shielded ($35) — modest upgrade с заметным noise improvement.

- Optional: external linear PSU as add-on accessory ($150 separate purchase).

- Subtotal: $22 BOM upgrade.

## Elite SKU total BOM расчёт

| Category | Budget baseline | Elite tier upgrade | Δ BOM |
| --- | --- | --- | --- |
| Op-amps | TL07x (~$2 total) | OPA1612 + Sparkos SS3601 | +$130 |
| JFETs | LSK489A $6 | LSK489B selected $10 | +$4 |
| OTAs | LM13700 ×2-3 ($4-6) | THAT2180 + LM13700 | +$6 |
| Caps (signal + EQ + bulk + decoupling) | $5 | Mundorf + Cornell + Elna + WIMA FKP | +$95 |
| Resistors | Standard 1% MF (~$3) | Dale CMF + Audio Note tantalum critical | +$25 |
| Pots & sliders | $14 baseline | Alps RK27 + RK09L + premium sliders | +$222 |
| Switches & footswitches | $14 | Carling sealed + Schurter | +$84 |
| Jacks (audio + CV) | $32 baseline | Neutrik gold + Thonkiconn gold + Switchcraft | +$72 |
| Cartridge connectors | $12 | Lemo PIB | +$96 |
| Cables | $1 | Mogami W2549 | +$2 |
| Exciter | $20 (DAEX25) | Visaton EX 60 S | +$30 |
| Piezo | $1 ×2 | K&K Pure + PVDF | +$45 |
| PCB | $6 (2-layer) | 6-layer ENIG immersion silver | +$90 |
| Enclosure | $15 | Solid copper corpus | +$285 |
| Knobs | $10 (20× $0.50) | Brass + Bakelite mixed | +$490 |
| DC-DC | $13 | Recom shielded REC15-1212S | +$22 |
| Subtotal upgrades |  |  | +$1,698 |

Elite SKU total BOM:

- Budget baseline: ~$94.

- +$1,698 upgrades.

- = ~$1,792 total BOM per Elite pedal.

Hold on — это слишком высокий. Realistic Elite SKU не должен blow всё на каждую категорию. Selective approach:

### Curated Elite SKU (sensible upgrades only)

| Selective upgrades (only where audible / visible) | Δ BOM |
| --- | --- |
| OPA1612 для signal path (skip Sparkos discrete extreme) | +$30 |
| LSK489B selected JFET | +$4 |
| THAT2180 для VCA only | +$6 |
| WIMA MKP signal caps + Nichicon Muse C_DC + matched silver mica EQ | +$30 |
| Critical resistors Dale CMF | +$3 |
| Alps RK27 для 6 big knobs only, RK09L для остальных | +$170 |
| Carling sealed footswitches | +$72 |
| Neutrik gold + Switchcraft gold jacks | +$40 |
| Lemo cartridge connectors | +$96 |
| Visaton EX 60 S exciter | +$30 |
| K&K Pure Mini + PVDF piezo | +$45 |
| 4-layer ENIG PCB (not 6-layer) | +$50 |
| Copper corpus | +$285 |
| Brass + Bakelite knob mix (top tier visual) | +$490 |
| Recom shielded DC-DC | +$22 |
| Curated Elite total Δ | +$1,373 |

Curated Elite BOM: ~$94 + 1,373=** 1,467**.

### Suggested Retail

| Tier | BOM | Retail | Gross margin |
| --- | --- | --- | --- |
| Budget | $94 | $499 | 81% |
| Premium | $121 | $649 | 81% |
| Elite (curated) | $1,467 | $2,499 | 41% |
| Elite (ultra, всё-всё-всё) | $1,792 | $2,999 | 40% |

Elite tier — lower margin, но absolute profit higher ($1,032 vs $405 на Premium). Limited edition strategy: 25 Elite SKU экземпляров in production, all serialized + provenance certificate.

## Sourcing matrix

### По категориям — где доставать

| Category | Primary supplier | Backup | Notes |
| --- | --- | --- | --- |
| Op-amps (TI/TI premium) | Mouser, Digi-Key | Newark, Octopart aggregator | Same-day shipping, paypal/credit |
| OPA627 vintage | eBay (verified sellers) | Audio forum classifieds (DIY Audio, Head-Fi) | Counterfeit risk — buy from known sellers only |
| Sparkos / Burson modules | Sparkos Labs direct (sparkoslabs.com) | Burson Audio direct | Long lead time (2-4 weeks) |
| JFETs (LSK489) | Linear Integrated Systems direct (linearsystems.com) | Mouser, Digi-Key | Order in batches |
| 2SK170 NOS | Japanese audio surplus (sayama-jhoji, Yahoo Japan Auctions) | eBay verified | Authentication critical |
| THAT2180 | Mouser, Digi-Key | THAT Corp direct | Stock usually OK |
| SSI2164 | Sound Semiconductor direct | Thonk, Modular Addict | Modular synth favorite |
| WIMA caps (MKS2/MKP) | Mouser, Digi-Key | RS Components (EU) | German precision, common stock |
| Mundorf Supreme | Mundorf direct (mundorf.com) | Parts Connexion (Canada), Sonic Craft (US) | Limited stock per batch |
| Auricap XO | Audience direct (audience-av.com) | Parts Connexion | US-made |
| Cornell Dubilier silver mica | Mouser, Allied Electronics | eBay (verified vintage radio sellers) | NOS available |
| Elna Silmic II / Nichicon Muse | Banzai Music (Germany) | Smallbear Electronics | Audio electrolytic specialists |
| Black Gate (NOS) | eBay, Japanese audio forums | Audio Note UK | Very rare, $50+ per cap |
| Dale resistors (CMF55) | Mouser, Digi-Key | Newark | Standard stock |
| Vishay precision (Z-foil) | Mouser, Digi-Key | Newark | Engineering elite |
| Audio Note tantalum | Audio Note UK direct (audionote.co.uk) | Parts Connexion | Audio Note dealer network |
| Alps RK27 "Blue Velvet" | Mouser, Smallbear, Thonk (UK) | Bourns (alternative) | Reliable stock, $25 each |
| Alps RK09L | Mouser, Smallbear | Banzai Music | Common stock |
| TKD CP-2500 | Park Audio (Japan, parkaudio.jp) | Specialty Japanese audio | Long lead time, expensive |
| Khozmo stepped attenuators | Khozmo direct (khozmo.com, Poland) | DIY audio forums | Hand-built, 2-4 week lead |
| Carling Technologies switches | Mouser, Digi-Key | Cliff Electronics direct (cliffuk.co.uk) | UK distributor |
| Schurter switches | Mouser, Digi-Key | Schurter direct | Swiss precision |
| Otax soft-touch footswitches | Free The Tone direct (Japan) | DIY pedal forums | Niche, premium feel |
| Neutrik jacks (gold) | Neutrik direct | Thonk, Mouser | Industry standard |
| Switchcraft jacks | Mouser, Digi-Key | Smallbear | American standard |
| Lemo connectors | Lemo direct (lemo.com) | DigiKey | Authorized distributor account preferred |
| Hirose connectors | DigiKey, Mouser | Japanese distributors | Excellent stock |
| Mogami / Canare cables | Mogami direct (US importer Audio Cables Inc), Canare direct | Sound Pure, Sweetwater Pro | Industry standard |
| Furutech cables | Furutech direct (furutech.com, Taiwan) | Audiophile dealers (Acoustic Sounds, Parts Connexion) | Japanese audiophile cult |
| Visaton exciter | Visaton direct (Germany) | Solen, Parts Express | Common in EU |
| Tang Band custom exciter | Tang Band direct (Taiwan) | Parts Express | Commission for custom voice coils |
| K&K Pure Mini pickup | K&K Sound direct (kksound.com) | Reverb.com | Niche acoustic pickup market |
| PVDF film LDT0-028K | Measurement Specialties direct | Mouser | Specialty film piezo |
| Schertler pickups | Schertler direct (Swiss) | Specialty acoustic luthier shops | Premium acoustic pickup |
| PCB 4-layer ENIG / 6-layer | JLCPCB (China), PCBWay | OSH Park (US small batch), Aisler (EU) | JLCPCB best price/quality |
| Rogers substrate PCB | Sunstone Circuits (US) | PCB Universe | RF-grade |
| Anodized aluminum corpus | Front Panel Express (US/EU) | Schaeffer / German Panel | Custom CNC |
| CNC 6061-T6 enclosure | Protocase (Canada), Sendcutsend (US) | Local CNC shop | Custom dimensions |
| Brass / copper enclosure | Custom metalshop / commission (etsy, eBay specialty) | Local copper / brass shops | One-off, expensive |
| Davies 1900H knobs | Selco Davies (US) | Smallbear, Mouser | Classic synth knob |
| Re'an knobs | Mouser, Digi-Key | Smallbear | OK budget premium |
| Bakelite knobs (custom) | Etsy artisans, eBay vintage radio | Specialty restorers | Lookup specific listings |
| Antique radio knobs (NOS) | eBay vintage radio category | Antique radio shows | Random availability |
| Solid brass knobs | Etsy custom machinist commissions | Mod kit suppliers | Custom-order |
| TRACO / Recom DC-DC | Mouser, Digi-Key | RS Components | Industry stock |
| Linear PSU audiophile | Mojo Audio, Cardas direct | Audiogon (used market) | Boutique audio |

## Risk register

| Risk | Severity | Mitigation |
| --- | --- | --- |
| OPA627 counterfeits (eBay) | HIGH | Buy only from Mouser/DigiKey official channels, never bulk eBay |
| 2SK170 NOS depletion | HIGH | Pre-buy stock, lock 100+ units now; design fallback к LSK170A |
| Mundorf Supreme out-of-stock | MEDIUM | Order 6+ months ahead, Mundorf produces in batches |
| Black Gate caps unavailable | EXTREME | Already discontinued ~2006. Substitute Elna Silmic II или Nichicon KZ |
| Lemo connector lead time | MEDIUM | 4-8 weeks typical, plan ahead |
| Custom copper corpus failures | MEDIUM | Identify reliable metal shop, do prototype first |
| K&K Pure Mini supply | LOW | K&K Sound stable, sometimes 2-week backorder |
| TKD pots discontinued | MEDIUM-HIGH | TKD has reduced product line; Alps RK27 reliable substitute |
| Audio Note resistors price spike | LOW | Already premium, price stable |
| Cartridge materials (Hetian nephrite) | HIGH | Source verification, ethical sourcing concerns. See cartridges/07_nephrite_processing.md |

## Recommended Phase 1 strategy

Не запускать Elite SKU сразу. Phase 1 ship = Budget + Premium tiers only. Reasons:

- Elite tier requires established customer base ($2,499 retail won't sell к unknown brand).

- Premium parts have long lead times — supply chain не tested.

- Premium audio claims need measured proof (THD/SNR) — testing infrastructure required first.

Elite SKU launch strategy:

- Phase 1: Budget + Premium ship, build brand recognition.

- Phase 2 (12+ months later): Open pre-order Elite tier with limited 25 units, deposit 50%.

- Phase 2B: Source components batch, build 25 Elite pedals, ship serialized + provenance certificate.

- Phase 3: Optional Elite tier для каждой production cycle (10/year ongoing).

### Provenance certificate (per Elite unit)

Включается с pedal:

- Serial number + production date.

- Component list с part numbers, suppliers, batch IDs.

- Hand-builder signature.

- Burn-in test report (24h continuous operation log).

- Photo of builder's workshop.

- Plate audio test recording (sweep + impulse response) on USB-thumb.

This becomes owner heirloom, justifies $2,499 price tier.

## Open questions

- Final Elite SKU BOM — нужно prototype + audition при первой сборке. Не все upgrades audible — A/B test required.

- Marketing positioning Elite — "limited edition art-object" vs "ultimate audiophile reverb"? Both possible.

- Provenance certificate platform — paper booklet + USB thumb или digital QR-coded online provenance page?

- Used parts policy — vintage Black Gate caps (NOS rare) — can use в Elite или only new components allowed?

- Cartridge tier coupling — Elite pedal ships с какой cartridge default? Recommend Hetian nephrite white или mammoth ivory limited edition.