# LAST DAY — Спецификация продукта

Версия: v1.0 (consolidated base — Decisions 02 + 08) Статус: R&D Phase A (prototyping) Window: Phase 2 ship per Decision 08 roadmap (12–24 месяца после Last Night Phase 1).

Last Day — постапокалипсис-делей раскалённого полудня. Парный модуль к Last Night (холодная ночь). Oil-can delay + solar saturator + tongue resonator + горячая палитра character FX (HAZE/MIRAGE/BLEACH/TAR/CICADA/HEATWAVE) + perform жесты (DRAG/CRASH/KILL/FREEZE).

## Содержание

- Обзор

- Архитектура и signal flow

- Контролы и интерфейс

- Картриджи — общий стандарт с Last Night

- Тембральный характер

- Связь с Last Night (диптих)

- Phase plan и production roadmap

- Открытые вопросы

## Обзор

### Что это

Last Day — electromechanical delay + tone shaping combine на oil-can технологии (Tel-Ray inheritance, capacitive pickup на conductive oil dielectric). Solar amp как oil-can master saturator (LED baseline + opt. solar cell для outdoor performance). Inductive shelf EQ + physical tongue resonator (manual clamp v1, motorized v2 premium) для midrange character. Поверх — hot palette FX layer (HAZE/MIRAGE/BLEACH/TAR/CICADA/HEATWAVE/OLD VINYL) и 4 perform footswitches (KILL/FREEZE/DRAG/CRASH).

### Чем отличается от обычных pedal-delay

- Реальный oil-can pickup: conductive mineral oil как dielectric, capacitive sensing — не PT2399 emulation, не BBD, не digital DSP. Audio-rate analog charge transfer через rotating disk + electrodes.

- Tongue resonator с физическим caподастром: пользователь двигает clamp вдоль guide rail → меняет резонансную частоту mid-range. Q control через clamp pressure. Acoustic plate-tongue resonator wrapped в audio feedback loop.

- Solar saturator: solar cell + internal LED → starved supply для opening stage. Outdoor performance = sun-driven saturation. Indoor = LED baseline stable.

- Hot palette FX antiподы Last Night cold palette: каждый Last Day эффект — semantic mirror Last Night эффекта (HAZE↔PULSE, BLEACH↔FROST, TAR↔CHILL, CICADA↔GEIGER, HEATWAVE↔HUM, DRAG↔STALL, CRASH↔TOLL).

- Cartridge стандарт един с Last Night — те же mini-XLR + JST connectors, те же мехи retention. Можно swap-картриджи между модулями (один материал слышен и как plate-reverb, и как oil-can wet-tap delay material).

### Категория

Постапокалипсис-делей с физическим механическим резонатором.

Pedal-format конкуренты (closest analogue): Hologram Microcosm, Chase Bliss MOOD MkII, Strymon Volante, Catalinbread Belle Epoch Deluxe.

Но никто из них не использует физическую oil bath с capacitive sensing + physical tongue resonator + solar saturation. Last Day — категория из одного.

### Reality check (для customer expectations)

- Oil-can delay — не "clean digital delay". Bandwidth 200Hz–4kHz (limited by motor speed jitter + disk surface variation). Sound = hazy, midrange-heavy, gentle pitch wobble (~±10 cents natural wow + motor LFO).

- Repeat times: 60ms (motor fast) — 800ms (motor slow). Beyond — feedback regenerates artefacts.

- Tongue resonator — добавляет distinct mid bump (Q ≈ 5), sweepable 200Hz–2kHz (full clamp range). Не precision EQ — это physical resonance с harmonic series смешанная с fundamental.

- Solar amp — outdoor: ярко, ясно → больше gain, plate saturation. Indoor: LED baseline, default voltage. Не unstable — opt circuit ensures graceful fallback.

- Service life: oil может потерять conductivity через 12–24 месяца (зависит от additive). Replaceable oil reservoir (sealed cartridge service). Motor brush wear ~5000 hours (premium SKU = brushless motor option).

### Use cases

- Dub / dub-techno — oil-can wow/flutter + FREEZE + DRAG для traditional dub gestures, KILL для silence drops.

- Drone / ambient — tongue resonator sweepable + TAR compression → infinite molasses sustain. CICADA добавляет insect-cluster texture над drone bed.

- Post-rock / shoegaze — HAZE wow + BLEACH high saturation → cassette-tape-like character на guitar. Не digital ambience — physical.

- Modular weirdo — patch CV в HAZE rate, BLEACH threshold, motor brake (DRAG CV input) → motor-speed-controlled spectrum manipulation.

- Performance / live — CRASH solenoid impact (oil splash → hydrophone) для percussive moment, DRAG для pitch-drop transitions.

## Эффекты "горячей палитры" (hot-palette FX layer)

Согласно Decision 08 mapping:

| Эффект | Knob/Switch | Реализация | Антипод (Night) |
| --- | --- | --- | --- |
| HAZE | Knob (lin) | Periodic LFO 0.05–5 Гц → motor PWM duty → speed mod → wow/flutter. | PULSE |
| MIRAGE | Knob (lin) | Aperiodic pitch drift ±10 центов через noise → S&H → slew → motor mod. | FOG |
| BLEACH | Knob (log) | HF saturation 4–12 кГц с solar-driven threshold. Soft-clip diodes. | FROST |
| TAR | Knob (log) | Peak-hold compressor с long-attack залипанием ~500 мс. Anti-CHILL. | CHILL |
| CICADA | Knob (lin) | Cluster tick generator: noise → env follower → comparator → retrigger bursts. Signal-correlated плотность. | GEIGER (cold half) |
| HEATWAVE | Knob (log) | AM-tuner front-end (TA7642) + ferrite antenna → detector → mix. Голос мёртвой ионосферы. | HUM |
| OLD VINYL | Knob (lo-fi char) | PT2399 в parallel с oil-can magnetic tract — добавляет grunge/bitcrush к delay tail. | Phaser (опционально) |
| DRAG | Footswitch (momentary) | MOSFET shorts motor → pitch уходит вниз, decay fades to silence. | STALL |
| CRASH | Footswitch (momentary) | Solenoid удар по oil bath → splash в delay через hydrophone. | TOLL |
| KILL | Footswitch (momentary) | Input mute. | KILL (общий gesture) |
| FREEZE | Footswitch (latching) | Erase head disabled → content loops infinitely, slowly degrades. | FREEZE (общий gesture) |

## What's in the box

- Last Day module (Eurorack 40HP или big-box pedal 203×140мм — две SKU).

- 1 starter cartridge: Disk Capacitive Oil-can (default v1 cartridge — sealed acrylic disk + 60ml conductive mineral oil + capacitive sensing electrodes).

- 12V DC PSU (pedal SKU): 12V center-negative, 800mA min (regulated).

- USB stick: SPEC.docx, BUILD.docx, oil-refill procedure manual.

- Knurled-knob set (×7 hot palette + ×4 master + ×3 EQ + 4 footswitches).

- Spare disk surface (replacement, для prolonged service).

- 1× oil refill bottle (60ml conductive mineral oil + additive).

### Compatibility — Eurorack ↔ Pedal

Same PCB, two enclosures.

- Eurorack SKU: 40HP × 128.5мм, ±12V Eurorack power, large performance buttons вместо footswitches.

- Pedal SKU: 203×140мм big-box (matches Strymon BigSky / Eventide H9 Max класс), 12V DC center-negative, isolated DC-DC → ±12V audio rails (same headroom как Eurorack).

I/O identical: TRS stereo + RCA stereo параллельно + 3.5мм mini-jack patch bay для CV (15 jacks Eurorack / 21 jacks pedal).

## Архитектура и signal flow

### High-level signal chain

Input (instrument / line / +4dBu)
        │
        ▼
   ┌──── Input buffer (TL072 ×1) ──── Input level knob ────┐
   │                                                        │
   │  Dry split tap ──► to dry mix bus (Block "MIX")        │
   │                                                        │
   └──► Solar amp / starved opening stage ──┐
                                              │
                                              ▼
                                ┌──── Oil-can delay engine ─────┐
                                │                                │
                                │  Capacitive write electrode    │
                                │      ↓                          │
                                │  Rotating oil disk             │
                                │  (motor: 60–800 ms repeat)     │
                                │      ↓                          │
                                │  2× Capacitive read electrodes │
                                │      ↓                          │
                                │  L tap (early) + R tap (late) → stereo wet │
                                └────────────┼───────────────────┘
                                              │
                                              ▼
                                ┌─── Inductive shelf EQ + tongue resonator ───┐
                                │                                              │
                                │  Low shelf inductor (Hammond)               │
                                │  High shelf inductor                        │
                                │  Tongue resonator (physical plate-tongue    │
                                │   с clamp position pot OR encoder v2)        │
                                │                                              │
                                └──────────────┼────────────────────────────────┘
                                                │
                                                ▼
                                ┌─── Hot palette FX layer ───┐
                                │                            │
                                │  HAZE (motor mod)          │
                                │  MIRAGE (noise→S&H mod)    │
                                │  BLEACH (HF sat)           │
                                │  TAR (compressor)          │
                                │  CICADA (cluster ticks)    │
                                │  HEATWAVE (AM-tuner mix)   │
                                │  OLD VINYL (PT2399 parallel)│
                                │                            │
                                └──────────────┼─────────────┘
                                                │
                                                ▼
                                ┌─── Feedback loop ───┐
                                │  RV_FEEDBACK (regen)│
                                │  Soft-clip limiter  │
                                │  (1N4148 diodes)    │
                                └──────────┼──────────┘
                                            │
                                            ▼
                                ┌─── Mix bus (TL074) ──┐
                                │  Dry ←── RV_DRY      │
                                │  Wet ←── RV_WET      │
                                │  CICADA/HEATWAVE/etc. ←── per-FX summing │
                                └──────────┼──────────┘
                                            │
                                            ▼
                                ┌─── Output buffer + perform gates ──┐
                                │  CMOS switch для KILL              │
                                │  Erase disable для FREEZE          │
                                │  Motor MOSFET short для DRAG       │
                                │  CRASH solenoid trigger            │
                                └──────────┼──────────────────────────┘
                                            │
                                            ▼
                                       Output L + R

### Что происходит звуку (по блокам)

Solar amp:

- Class A discrete BJT amp с starved supply (3–8 В variable).

- Solar cell + photoresistor → control voltage → opt rail.

- Outdoor: bright light → high rail → low compression → clean amp.

- Indoor: LED baseline → fixed ~5V rail → mild compression.

- Saturation = harmonic enrichment + asymmetric distortion (BJT-friendly).

Oil-can engine:

- Disk speed controlled by PWM-driven DC motor (BLDC opt premium SKU).

- Motor speed: 200 RPM (slow, 800ms repeat) → 2000 RPM (fast, 60ms repeat).

- Capacitive pickup: write electrode + 2 read electrodes 90°/180° offsets → L/R stereo taps.

- Modulation paths: HAZE adds periodic LFO к motor PWM, MIRAGE adds aperiodic S&H, DRAG (footswitch) shorts motor to GND → coasts to stop.

Tongue resonator:

- Physical metal tongue strip 80×8×0.4mm bolted в slot.

- Clamp moves along strip via lead screw → effective tongue length changes → resonant frequency changes.

- Manual v1: panel knob rotates lead screw via PTFE flexible coupling.

- Motorized v2: stepper motor + encoder → CV position recall.

- Q: tongue clamp pressure (knob adjusts spring tension on clamp jaw).

- Sweep range: 200Hz (full extension) — 2kHz (clamp at base).

Hot palette FX: каждый эффект — opt layer over delay output. Per-FX VCAs allow mute/blend.

## Контролы и интерфейс

### Раскладка панели — Pedal SKU (203×140мм, big-box class)

┌───────────────────────────────────────────────────────────────┐
│ LAST DAY — postapocalypse oil-can delay combine              │
│                                                                │
│   ┌──────────┐  ┌──────────────────────────┐  ┌──────────┐   │
│   │ MOTOR    │  │  TIME · FEEDBACK · MIX   │  │ CARTRIDGE│   │
│   │ SPEED    │  │                          │  │  SLOT    │   │
│   │ slider   │  │  (top row: master)       │  │ (oil-can │   │
│   │ 1P5T     │  │                          │  │  disk    │   │
│   │ presets  │  │  ⊙Time  ⊙FB    ⊙Mix     │  │  visible)│   │
│   └──────────┘  └──────────────────────────┘  └──────────┘   │
│                                                                │
│   Row A: ⊙HAZE  ⊙MIRAGE  ⊙BLEACH  ⊙TAR  ⊙CICADA  ⊙HEATWAVE   │
│   Row B: ⊙OLD VINYL  ⊙LO SHELF  ⊙HI SHELF  ⊙TONGUE  ⊙Q  ⊙Solar│
│   Row C CV bay: ⊙Time  ⊙FB  ⊙Speed  ⊙Mix  ⊙Tongue  ⊙TAP_IN   │
│   Row D CV bay: ⊙HAZE  ⊙MIRAGE  ⊙BLEACH  ⊙TAR  ⊙CICADA  ⊙HEAT │
│                                                                │
│   ┌─────────────────────────────────────────┐                  │
│   │  Solar cell window (5×3 см на лицевой)  │                  │
│   │  + internal LED behind diffuser         │                  │
│   └─────────────────────────────────────────┘                  │
│                                                                │
│   Tongue resonator slot (visible tongue strip):                │
│   ┌─────────────────────────────────────────┐                  │
│   │  ╔═══════════════════════════════════╗  │                  │
│   │  ║ Tongue strip + clamp position bar ║  │                  │
│   │  ╚═══════════════════════════════════╝  │                  │
│   └─────────────────────────────────────────┘                  │
│                                                                │
│   Footswitches (bottom row):                                   │
│   [KILL]  [FREEZE]  [DRAG]  [CRASH]                            │
│                                                                │
│   Audio I/O (back panel): IN (TRS/RCA), OUT L+R (TRS/RCA),     │
│   12V DC jack, expression pedal jack                            │
└───────────────────────────────────────────────────────────────┘

MOTOR SPEED slider 1P5T (5 положений) — preset speed для disk motor:

- Pos 1: SLOW (60 RPM → ~800ms repeat)

- Pos 2: MEDIUM-SLOW (250 RPM → ~480ms)

- Pos 3: MEDIUM (650 RPM → ~250ms)

- Pos 4: FAST (1200 RPM → ~140ms)

- Pos 5: WIRE (2000 RPM → ~60ms, doubles as "chorus mode")

Continuous control via ⊙Time + CV ⊙Time для fine adjustment.

### Раскладка панели — Eurorack SKU (40HP × 128.5мм)

Та же layout, но:

- Footswitches заменены на large performance buttons (push-momentary, 12 мм tall).

- CV bay denser (15 mini-jacks вместо 21 у pedal — pedal duplicates some jacks как RCA/TRS на back).

- Solar cell window обязателен (open panel face for outdoor performance).

### Полная таблица контролов

| Группа | Контрол | Тип | Диапазон / функция |
| --- | --- | --- | --- |
| Master | ⊙Time | 100kΩ log | Motor speed (60ms–800ms repeat) |
|  | ⊙FB | 100kΩ lin | Feedback regen 0–95% |
|  | ⊙Mix | 100kΩ lin (center detent) | Dry/wet blend |
| Motor presets | SL-1P5T slider | 5-position | SLOW / M-SLOW / M / FAST / WIRE |
| Hot palette | ⊙HAZE | 100kΩ lin (center detent) | Periodic motor mod depth |
|  | ⊙MIRAGE | 100kΩ lin | Aperiodic pitch drift depth |
|  | ⊙BLEACH | 100kΩ log | HF saturation amount |
|  | ⊙TAR | 100kΩ log | Compressor sustain |
|  | ⊙CICADA | 100kΩ lin | Cluster tick density |
|  | ⊙HEATWAVE | 100kΩ log | AM-tuner mix level |
|  | ⊙OLD VINYL | 100kΩ lin | PT2399 parallel grunge |
| EQ + resonator | ⊙LO SHELF | 50kΩ lin (cut/boost ±12dB @ 150Hz) | Inductor low shelf |
|  | ⊙HI SHELF | 50kΩ lin (±12dB @ 4kHz) | Inductor high shelf |
|  | ⊙TONGUE | 10kΩ lin (clamp position via lead screw) | 200Hz–2kHz resonant freq |
|  | ⊙Q | 100kΩ lin (clamp pressure → spring tension) | Q 1–8 |
| Saturator | ⊙Solar | 100kΩ log | Internal LED brightness (+ solar override) |
| Switches | SW_INDOOR | SPDT | LED-only / Solar-enabled |
|  | SW_REGION | DPDT slide | 50Hz / 60Hz (for HEATWAVE tuning baseline) |
| Footswitches | KILL | momentary 3PDT | Input mute |
|  | FREEZE | latching 3PDT | Erase head disable → loop |
|  | DRAG | momentary 3PDT | Motor brake (pitch fall) |
|  | CRASH | momentary 3PDT | Solenoid oil-splash impact |

### CV Patch Bay (Pedal: ~21 jacks; Eurorack: ~15 jacks)

Audio I/O:

- IN (mono TRS + dual RCA в parallel)

- OUT L, OUT R (stereo TRS + dual RCA)

- DRY OUT, WET OUT (separate post-buffer taps)

CV Inputs (Phase 1):

- IN level CV

- Time CV (motor speed CV → ±5V → speed mod)

- FB CV

- Speed CV (alias of Time CV для clarity на panel)

- Mix CV

- Tongue position CV (CV → stepper motor v2 premium только)

- TAP_IN gate (TAP sync motor speed к beat)

- HAZE / MIRAGE / BLEACH / TAR / CICADA / HEATWAVE CV (per-FX modulation)

- Solar CV (override solar voltage from external)

Gate / trigger inputs (modular advanced):

- KILL_TRIG

- FREEZE_TRIG

- DRAG_TRIG (gate → motor brake duration)

- CRASH_TRIG

### Cartridge connector (внутри slot, идентичен Last Night)

- 2× mini-XLR (Switchcraft TA3F) — capacitive read electrodes A + B (stereo wet taps).

- 2× JST-XH 2-pin — disk motor power + write electrode HV bias (~30V).

- 4× neodym магниты для retention.

- 1× retention pin (anti-pull, springloaded).

## Картриджи — общий стандарт с Last Night

Cartridge frame identical: 110×65×30мм PETG (production: anodized aluminium). Mini-XLR + JST соответствуют Last Night.

Cartridge различия по содержанию:

| Cartridge type | Content | Use в Last Night | Use в Last Day |
| --- | --- | --- | --- |
| Material plate | Wood/stone/metal/glass plate + exciter + dual piezo | Plate reverb resonator | Wet tap acoustic body (signal через physical plate as series resonator перед oil-can) |
| Oil-can disk | Sealed acrylic disk + 60ml conductive oil + capacitive electrodes | Wet tap acoustic body (delay sub за reverb tail) | Primary delay engine |
| Premium PVDF plate | Plate + PVDF pickup | Plate reverb premium | High-precision wet acoustic body |

Key insight: один cartridge стандарт служит обоим модулям. User может вставить material plate в Last Day → получает plate-as-pre-delay-resonator (тот же материал, но как mid-stage filter, не reverb). И наоборот — oil-can disk в Last Night → tap delay sub за reverb tail.

### Картридж swap procedure

Идентична Last Night:

- Power off module.

- Push retention pin → cartridge releases.

- Pull cartridge straight out (mini-XLR + JST auto-disconnect via spring).

- Insert new cartridge → push в slot → retention pin auto-locks.

- Power on.

Hot-swap не рекомендуется (потенциальный pop на capacitive write electrode при breaking +30V HV bias path).

## Тембральный характер

### Общий character

- Hazy: midrange-heavy, slight pitch wobble (~±10 центов natural disk surface variation), bandwidth 200Hz–4kHz typical.

- Organic: каждый "repeat" чуть-чуть different — disk surface не идеальна, capacitive sensing meander с small variations.

- Warm: solar amp adds harmonic richness (even harmonics dominate), inductor EQ adds gentle phase shift.

- Alive: tongue resonator буквально звенит в hands (small Q ≈ 5, but audible в acoustic body of pedal).

### По регистру

- Bass: oil-can clips bass naturally (motor speed wow exaggerates на низах, < 100Hz unstable). LO SHELF позволяет boost up to ±12dB @ 150Hz.

- Mid: tongue resonator + inductor shelf даёт rich midrange character.

- Highs: limited by capacitive pickup bandwidth (~4kHz natural roll-off). BLEACH saturator восстанавливает HF через harmonic generation.

## Связь с Last Night (диптих)

Согласно Decision 08 symmetric mapping:

| Cold (Last Night) | ↔ | Hot (Last Day) | Phenomenon |
| --- | --- | --- | --- |
| Phaser (immersion) | ↔ | OLD VINYL (PT2399 parallel) | Tape/vinyl decay character |
| PULSE | ↔ | HAZE | Periodic modulation |
| FOG | ↔ | MIRAGE | Aperiodic drift |
| FROST | ↔ | BLEACH | HF subtract ↔ add |
| CHILL | ↔ | TAR | Expander ↔ compressor |
| GEIGER | ↔ | CICADA | Sparse ticks ↔ cluster ticks |
| HUM | ↔ | HEATWAVE | Mains hum ↔ AM tuner ионосфера |
| STALL | ↔ | DRAG | Held damper ↔ motor brake |
| TOLL | ↔ | CRASH | Strike plate ↔ strike oil |
| KILL | ↔ | KILL | Common gesture |
| FREEZE | ↔ | FREEZE | Common gesture |

Patch-pair use cases:

- Set Last Night tail → tap of tail into Last Day input → Last Day delays the reverb → output combines into ambient bed.

- Cross-CV: Last Night solenoid TOLL gate → Last Day CRASH trigger → synchronized "bell + splash" gesture.

- Cartridge swap drama: material plate из Night → Day mid-stage (or vice versa) → instrument has TWO acoustic bodies in series.

## Phase plan

Per Decision 08 consolidated roadmap (Last Day track):

| Phase | Window | Deliverable |
| --- | --- | --- |
| Phase A (months 1–12) | R&D ongoing | Prototype motor + capacitive pickup + oil bath sealed cartridge + inductive EQ + manual tongue resonator + solar amp с LED baseline. v1 ship: KILL + FREEZE + DRAG + HAZE (4 base perform FX) + LO/HI shelf + tongue Q + Time/FB/Mix master + Solar. |
| Phase B (months 12–18, ~9 после Last Night ship) | Hot palette v1 | Add MIRAGE / BLEACH / TAR / CICADA / HEATWAVE / OLD VINYL / CRASH. Full hot palette implementation. v1.5 ship. |
| Phase B2 | months 18–27 | Motorized tongue resonator с encoder + CV-recall preset (v2 premium SKU). |
| Phase C (months 27+) | Boutique tier | Premium oil formulations (graphene-doped, ionic liquid alternatives). BLDC motor option (low-jitter). Solid copper corpus. Aligned с Last Night Decision 10 Elite tier. |

v1 ship timeline: ~12 месяцев после Last Night Phase 1 ship. Last Day v1 release window: months 21–27 после project kickoff.

## Открытые вопросы

- Oil composition — conductive mineral oil с graphite additive (cheapest) vs ionic liquid alternative (30/100мл)vspremiumgraphene−doped(100/100мл). Decision на BOM cost vs longevity tradeoff.

- Motor selection — brushed DC (3,5000hlife)vsBLDC(15, 30000h life). Premium SKU recommendation: BLDC. Budget SKU: brushed.

- Solar cell sourcing — small format (40×30мм) от Mouser (2–8)vscustomtransparentencapsulated(20+). Decision на aesthetic vs cost.

- PT2399 OLD VINYL parallel tract — wet level dominates how much grunge bleeds в clean delay tap. Default mix level?

- HEATWAVE tuning baseline — region switch 50Hz/60Hz primarily для AM-tuner reference voltage, but tuning capacitor range needs final spec. 535–1605 kHz AM band is target.

- CRASH hydrophone — piezo disk submerged в oil bath. Cable feedthrough requires hermetic sealing (что не trivial при cartridge swap). Alternative: external hydrophone в separate sealed oil pocket?

- Tongue resonator Q — clamp pressure control via knob — needs механический prototype to validate that user-friendly Q range (1–8) achievable без torque-amplifier mechanism.

- Production strategy — Day shares ~60% PCB с Night, ~40% Day-specific (motor driver, capacitive pickup analog frontend, inductor EQ network, solar amp). Single PCB design with depopulated variants vs two PCB designs?

## Документация

- LAST_DAY_BUILD.md — детальная build documentation (R&D Phase A — TBD после prototype completion).

- decisions/02_last_day_scope.md — D1–D6 decisions.

- decisions/08_consolidated_base.md — diptych consolidation, hot/cold palette mapping.

- SYSTEM_SUICIDE.md §10 — Last Day brief в series context.

Last Day = Last Night's hot twin. Same architecture (cartridge slot + FX engine + perform footswitches), different physical phenomenon (oil-can delay vs plate reverb), different temperature (raskali polden vs holodnaya noch). Together — диптих.